A Rapid Response Fluorescence on Sensor Synthesis, its Characterization and Application for Mercury Ion Recognition and Removal in Aqueous Media

Maria Sadia

Department of Chemistry, University of Malakand, Chakdara, Lower Dir, Khyber Pakhtunkhwa, Pakistan

Email: mariasadia@gmail.com

Abstract

In the present study, curcumin based ﬂuorescent sensor S for sensing of Hg2+ ion in aqueous samples was designed and characterized by UV-Visible, 1HNMR, and FTIR spectroscopy. The synthesized fluorescent probe S was successfully used for highly sensitive and selective fluorimetric determination of trace amount of Hg2+ ion in aqueous samples. Chelation-enhanced ﬂuorescence was shown by the fluorescent sensor S upon interaction with Hg2+ ion. This ﬂuorescent sensor S formed 2: 1 (S: M) complex as determined by Job’s plot analysis. A rapid response was observed when employed for the determination of Hg2+ ion in real aqueous samples with a limit of detection equal to 3.4×10− 9 M. Association constant was found to be 1×10 8 M −1 estimated from the Benesi-Hildebrand plot. No signiﬁcant interference occurred in the presence of other studied heavy metal ions.

Key words: Fluorescent sensor S, fluorimetric determination, aqueous samples, curcumin, mercury ion