One-Step Assembly of Organophosphorus Hydrolase and Affinity Peptide on Phage and Its Application on Biosensor

An-Yu Chen, Department of Chemical Engineering, National Taiwan University of Science and Technology, Taiwan

Cyuan-Jhe Wu, Department of Chemical Engineering, National Taiwan University of Science and Technology, Taiwan

Shen-Long Tsai, Department of Chemical Engineering, National Taiwan University of Science and Technology, Taiwan

Abstract

Organophosphorus compounds (OPs), a group of neurotoxins, are widely used as pesticides and insecticides. They not only pollute the environment but also deteriorate human health. Owing to their destructive effects, these compounds should be strictly regulated and monitored. Therefore, emphasis should be put on developing methods for immediate detection and effective elimination.

In our previous study, a novel biosensor utilizing Organophosphorus hydrolase (OPH) for OPs degradation and detection was proposed. M13 filamentous bacteriophage served as a vector carrying OPH on the surface of its pVIII coat protein via leucine zippers. Moreover, a mild steel-binding peptide was attached on its pIII coat protein for immobilization on mild steel electrode surface.

The activity of OPH/M13 phage/mild steel biosensor was determined via electrochemical analysis and spectrophotometer.

To our knowledge, the most important improvement is that the produced phage can be easily recovered by centrifugation rather than labor waste procedure. This results demonstrated the capability of proposed sensor may be useful in further development since both the specificity and activity of genetically engineered M13 phage are better compared to the wild-one.

Biography

An-Yu Chen graduated from National Tainan Girl’s Senior High School in June 2012, and from National Chung Cheng University which majored in Chemical Engineering in July 2016. She was interested in applications of biochemistry and materials during her college studies. Now she is a partner in the Cellular and Biomolecular Engineering Lab, National Taiwan University of Science and Technology, completing her Masters of Science degree in Chemical Engineering.

Email: w955488@gmail.com

Notes/Comments: