RP-HPLC METHOD FOR SIMULTANEOUS DETERMINATION OF SUMATRIPTAN SUCCINATE AND NAPROXEN SODIUM IN BULK AND TABLET DOSAGE FORM

Patel Pareshkumar Umedbhai, Shree S. K. Patel College of Pharmaceutical Education and Research, Ganpat University, Mehsana-Gozaria Highway, Ganpat Vidyanagar-384012, Dist: Mehsana, Gujarat, India

### Abstract

### A simple, accurate and rapid reversed phase high performance liquid chromatographic method has been developed and validated for the simultaneous estimation of Sumatriptan Succinate (SMT) and Naproxen Sodium (NAP) in bulk and tablet dosage form. The chromatographic separation was achieved on Purospher® Star C18 column (250 mm x 4.6 mm i.d., 5 µm), using mobile phase comprised of acetonitrile : 0.05% TFA in water (60:40, v/v), at a flow rate of 1.0 ml/min. The determination was carried out at 277 nm wavelength by PDA detector. The method was linear over a concentration range 5 - 80 μg/ml for both drugs. The retention time of Sumatriptan Succinate and Naproxen Sodium were found to be 2.247 min and 5.657 min respectively. Results of analysis were validated statistically and by recovery studies. The recovery was obtained in the range of 98.88 – 101.6 and 98.71 – 101.7 for SMT and NAP, respectively. The limit of detection (LOD) was found to be 0.67 μg/ml and 0.25 μg/ml, while the limit of quantification (LOQ) was 2.03 μg/ml and 0.77 μg/ml for SMT and NAP, respectively. The proposed RP-HPLC method was successfully applied for determination of SMT and NAP in their combined tablet dosage form. The percentage of SMT and NAP were found to be satisfactory; which was comparable with the corresponding label claim. The proposed method was found to be simple, accurate, precise, sensitive and robust. Hence, it can be used successfully for the routine analysis of SMT and NAP in pharmaceutical dosage form.

Recent Publications (minimum 5)

1.	Development and Validation of Simultaneous Equations Method for the Simultaneous Estimation of Telmisartan and Nifedipine in Synthetic Mixture, Modi Dixita V. and Patel Paresh. U., International Research Journal of Pharmacy, 2016, 7(5), 6-9.

2.  Development and Validation of High Performance Thin Layer Chromatographic Method for Simultaneous Estimation of Roflumilast and Salmeterol in Synthetic Mixture., Patel Ushma B. and Patel Paresh U., World Journal of Pharmaceutical Research., Volume 5, Issue 5, 2016., 1107-1117.

3.  Development and Validation of High Performance Thin Layer Chromatographic Method for Simultaneous Estimation of Temozolomide and Capecitabine in Synthetic Mixture., Pooja B. Patel and Paresh U. Patel., World Journal of Pharmaceutical Research., 2016, 5 (5), 1308-1316

4.  Development and Validation of Spectrophotometric Method for Simultaneous Determination of Temozolomide and Capecitabine in Synthetic Mixture., Pooja B. Patel and Paresh U. Patel., World Journal of Pharmaceutical Research, 2016, 5 (4), 1042-1050.

5. Development and Validation of Q-absorbance Ratio Method for Simultaneous Determination of Roflumilast and Salmeterol in Synthetic Mixture., Patel Ushma B. and Patel Paresh. U., International Research Journal of Pharmacy, 2016; 7(1), 58-61.

6.  Development and Validation of pH Independent Spectroscopic Method for Determination of Capecitabine, Shailja H. Patel and Paresh U. Patel, Pelagia Research Library, Der Pharmacia Sinica, 2015, 6(4):19-23

7. Simultaneous Estimation of Didanosine and Stavudin in Combination in Synthetic Mixture by RP-HPLC, Richa Dayaramani and Paresh U. Patel, Journal of Harmonized Research, 2015, 4(2), 189-200.

Biography (150 word limit)

Dr. Pareshkumar Umedbhai Patel has expertise in the field of Development and validation of analytical methods for estimation of drugs in bulk and in formulations, Validation of analytical methods and process, Documentation for regulatory affairs, Development of stability indicating analytical methods for drugs and pharmaceuticals, Standardization and evaluation of herbal formulations and products. This developed and validated method gives good results. So this method can be used in the routine analysis of drugs in combination in Quality Control.

Email: parashka@yahoo.com