Nonlinear optimization of the crane movement in the transient mode

Yuriy Romasevych, PhD, National University of Life and Environmental Sciences of Ukraine, Ukraine

Viatcheslav Loveikin, PhD, National University of Life and Environmental Sciences of Ukraine, Ukraine

Olexiy Stekhno, post-graduate student, National University of Life and Environmental Sciences of Ukraine, Ukraine

Statement of the Problem: Optimal control problems of tower, bridge and overhead cranes provide the basis for developing the effective control systems. Practical importance of optimal control problems is connected with increasing of cranes’ productivity, energy efficiency and reliability. A wide range of methods have been used in the studies of the optimal control problems: principle maximum [3, 4, 5], dynamical programming, controllability function method [2], reducing the original problem to the nonlinear programming problem [1]. One of the features of the optimal control problem is criterion. For instance, requirement of increase the energy efficiency leads to the necessity of utilizing the nonlinear criterion. Such problem has not been previously studied. The purpose of this study is to obtain optimal law of tower crane carriage motion according to nonlinear energy criterion. Methodology & Theoretical Orientation: In the investigation the two-mass dynamical model has been used (one mass represents the carriage, another – the cargo). The optimization criterion was root-mean-square power of the carriage drive during the start-up mode. The initial conditions of the system correspond to the rest state. The final conditions describe steady velocity of the masses with no oscillations of the cargo. The optimal control function was a scalar multiplication of the state vector and the vector of coefficients to find. In order to find them the modification of PSO technique was used. Findings: The plots, which have been built in the figure 1, clearly show that found optimal control transmits the system from origin (initial state) to the point of desirable final conditions and does not cross the boundary of the domain. Conclusion & Significance: Obtained results allows for decreasing the energy losses in the tower crane drive. In addition, the developed approach has clearly showed its efficiency for the complicated nonlinear problem solving.

a)

b)

Figure 1: Graphical interpretations of the results: a) phase portrait of the cargo oscillations; b) control function (black line) and trolley velocity (grey line)

Recent Publications

- Loveikin V.S., Romasevich Y.A., Khoroshun A.S., Shevchuk A.G. (2018) Time-Optimal Control of a Simple Pendulum with a Movable Pivot. Part 1. International Applied Mechanics, vol. 54, Issue 3: 358-365.

- Okun A., Los Y. (2016) The controllability function method” in UPB Scientific Bulletin, Series D: Mechanical Engineering, vol. 78, Issue 3: 3-8.

- Gerasimyak R.P., Leshchev V.A. (2008) Анализ и синтез крановых электромеханических систем (Analysis and Synthesis of Crane Electromechanical Systems). – Odessa: SMIL: 192.

- Ermidoro M. , Formentin S. , Cologni A., Previdi F., Savaresi M.S. (2014) On time-optimal anti-sway controller design for bridge cranes / 2014 American Control Conference (ACC): 2809-2814.

- He Chen,Yongchun Fang, Ning Sun. (2016) Optimal trajectory planning and tracking control method for overhead cranes / IET Control Theory & Applications., Vol. 10, Issue 6: 692-699.

Yuriy Romasevych has his expertise in solution of optimal control problems in different fields of engineering (control of lifting machines, synthesis of optimal automatic controllers, etc.). In his research he uses well-known methods of variational calculus, dynamic programming, principle maximum as well as new techniques which are based on swarm intelligent methods. His recent investigations are devoted to developed a few modifications of particle swarm optimization method and applied them to optimization of PI-controllers and robotic systems.

Email: romasevichyuriy@ukr.net

Viatcheslav Loveikin is a highly-skilled expert in machines dynamics, optimal control of lifting machines, robotic systems. His scientific methodology is based on the utilizing variational calculus, direct variational methods and other optimization techniques. Different statements of optimization problems allowed him to obtain effective results. Many of those have been implemented in control systems of tower, overhead and bridge cranes. Another field of research of the scientist is conveyors. Here he has obtained superior results, which related to increasing of their energy efficiency.

Email: lovvs@ukr.net

Olexiy Stekhno is carrying out investigations in the field of optimization of tower cranes. The aim of his research is to improve dynamic and energetic indicators of s tower crane as well as its productivity. The approach, which was developed by him, is based on the idea of complex optimization of parameters of a crane and its modes of motion.

Email: alexeystekhno1992@ukr.net