Successful rate of voice controlled switches in the noisy environment:  Original, tube, absorption cloth, and combination

Yuxin Li,  Zetian Chen,  Zide She,  Xin Bai, Undergraduate Students, GDUPT, China

Bing-Yuh Lu, Prof. PhD, GDUPT, China

Abstract (300 word limit)

Statement of the Problem: In a noisy environment, the successful rate (SR) of a voice controlled switches (VCS) is not easy to operate well. Therefore, we tried to add a tube, an absorption clothes, and their combination in front of the microphone of the voice recognition module for SR improvement. A voice recognition module connected with Arduino microcontroller which assigned the switches to turn on or off by the instructions of the user’s voice. The siren of ambulance is a stable voice and suitable for an example of noise. The experiment was designed by the stable voice (60~65 dB) over the variable noises (35~55 dB). The voices and siren sound was measured by a mobile application (APP) of Decibel tester.  The SNRs are:  (a) 20 to 30 dB; (b) 10 to 20 dB; (c) 5 to 15 dB and the color bars present the SRs (50 tests) of original condition (blue), adding a tube (red), adding an absorption cloth (green), and combination of adding tube and cloth (purple) in the following figure where H means the voice instruction of “Higher temperature”, “Lower temperature” and “Stop” in Chinese.  The findings of the SR of voice control presented that the SNR over 10 dB performs almost 100% SR. The normal human voices are around 60 dB, however, the distance between mouth and microphone should be considered. The common environmental noise in a house is in the range of 40 to 50 dB. Therefore, it is very easy to achieve the SNR>10 dB. The performance is close to the previous studies. Consequently, the system works in general cases.  Conclusion & Significance: This study attempted to use accessories for improving the SR. However, the performance is not obvious. Therefore, we will study to modify the shape of tube and the absorption cloth.

Image

- (b)                           (c)

The results of experiments: The voices are in the range of 60 to 65 dB.  The SNRs are:  (a) 20 to 30 dB; (b) 10 to 20 dB; (c) 5 to 15 dB and the color bars present the SRs (50 tests) of original condition (blue), adding a tube (red), adding an absorption cloth (green), and combination of adding tube and cloth (purple).

Recent Publications (minimum 5)

- Al-Haddad SAR, Samad SA, Hussain A, Ishak KA (2008) Isolated Malay digit recognition using pattern recognition fusion of dynamic time warping and hidden Markov models. American Journal of Applied Sciences, 5(6): 714-720.

- Fourcin A, Abberton E (2008) Hearing and phonetic criteria in voice measurement: Clinical applications. Logopedics Phoniatrics Vocology, 33(1): 35-48.

- Lu BY, Hsueh ML, Wu  HD (2017) Reducing the ambulance siren noise for distant auscultation of the lung sound. Acoustics Australia 45(2): 381-387.

- Website of Arduino, http://www.arduino.com.

- Website of DFRobot, https://www.dfrobot.com.

Yuxin Li is an undergraduate student of Guangdong University of Petrochemical Technology (GDPET), Guangdong, China .

Zetian Chen is an undergraduate student of GDPET, Guangdong, China.

Zide She is an undergraduate student of GDPET, Guangdong .China. His fields of interested include coding and control system. He was the winner of the coding competition in the Southern China in 2019.

Xin Bai is an undergraduate student of GDPET, Guangdong, China.

Bing-Yuh Lu* is a professor with Faculty of Automation, GDPET, Guangdong .China. He focused on the researches of biomedical engineering, applications of microprocessor, computer simulation and modelling. He is a member of IEEE and has served as a reviewer for some journals.

*Email: franklinlu888@outlook.com

Notes/Comments: