T Furuta et al., J Pai n Relief 2016, 5:3(Suppl) http://dx.doi.org/10.4172/2167-0846.C1.006

International Conference on

Fibromyalgia and Chronic Pain (June 15-16, 2016   Philadelphia,

In house cellulase production from mutant Aspergillus fumigatus FCC 01-01 strain and its application in bioethanol production using

lignocellulosic material

Abdul Sattar Qureshi, University of Sindh, Pakistan.

Imrana Khushk, University of Sindh, Pakistan.

Imran Ibrahim, University of Sindh, Pakistan.

Farah Naz Talpur, University of Sindh, Pakistan. Muhammad Asif, University of Sindh, Pakistan.

Abstract

Fuel ethanol has great and promising market values on replacing petroleum derived gasoline. Currently, cellulosic ethanol produced from lignocellulose biomass started its commercialization in USA and EU countries, but there still exist many technical difficulties to be solved to make process feasible for large scale commercialization. In our previous studies, high ethanol titer and high yield were achieved using evolutionary adapted Saccharomyces cerevisiae DQ1 for reduction of ethanol distillation energy cost. In addition, dry dilute acid pretreatment and biodetoxification was developed to help in commercialization of lignocellulosic biorefinery process.

Among various cost items in the biorefinery process from agricultural lignocellulose biomass, the cellulase enzyme cost used for saccharification of cellulose into glucose is considerably high in large scale commercial plants and the on-site cellulase production is preferred. In the present study, a mutant strain Aspergillus fumigatus FCC 0101 was evaluated for cellulase production in shake flasks. Optimization of cultural conditions enhanced 6 folds in cellulase activity to 3.2 FPU/mL after 5 days’ culture at 30 °C. Crude cellulase was characterized to obtain the optimal pH, temperature, as well as the pH stability and thermostability. Maximal activity was found at pH 5.5 and 50 °C. Enzyme retained 68 %, 53 % and 36 % activities after 48, 72 and 96 h incubation at 50 °C, respectively. Hydrolysis efficiency of the crude cellulase was comparable to the commercial cellulase enzyme Youtell #7. The results show the fermentable sugars released from hydrolysis of dilute acid pretreated sugarcane bagasse and wheat straw were 0.71 g/g and 0.57 g/g, respectively. 42 g/L of ethanol was obtained when the crude cellulase was applied in saccharification of sugarcane bagasse. Utilization of the on-site produced cellulase in bioethanol production saved the costs of enzyme purification, packing and transportation cost and consequently reduce ethanol production cost.

Image

Recent Publications (minimum 5)

- R.K. Sukumaran, R.R. Singhania, A. Pandey, Microbial cellulasesproduction:  applications and challenges, J. Sci. Ind. Res. 64 (2005) 832–844.

- R.R. Singhania, R.K. Sukumaran, A.K. Patel, C. Larroche, A. Pandey, Advancement and comparative profiles in the production technologies using solid-state and submerged fermentation for microbial cellulases, Enzyme Microb. Technol. 46 (2010) 541–549.

- R. R. Singhania, R. Saini, M. Adsul, J.K. Saini, A. Mathur,D. Tuli.  An integrative process for bio-ethanol production employing SSF produced cellulase without extraction. Biochem. Eng. J. 102 (2015) 45-48.

- M. Adsul, B. Sharma, R.R. Singhania, J.K. Saini, A. Sharma, A. Mathur, R. Gupta, D.K. Tuli, Blending of cellulolytic enzyme preparations from different fungal sources for improved cellulose hydrolysis by increasing synergism, RSC Adv. 4 (2014) 44726– 44732.

- He Y, Zhang L, Zhang J, Bao J. Helically agitated mixing in dry dilute acid pretreatment enhances the bioconversion of corn stover into ethanol. Biotechnol Biofuels 7 (2014), 7:1.

Abdul Sattar Qureshi has completed his Ph. D in 2015 from East China University of Science and Technology, Shanghai, China. H

e is Assistant Professor of

Biotechnology and Genetic Engineering in University of Sindh, Jamshoro, Pakistan. He has more than 14 years teaching and research experiences in Biotechnology. He has published 22 research papers and more than 10 are submitted in reputed journals. Dr. Sattar is reviewer of Applied Biochemistry and Biotechnology, Bioresource and Bioprocessing, African Journal of Biotechnology and many more.

Email: sattar.qureshi@usindh.edu.pk

J Pain Relief Fibromyalgia 2016 Volume 5, Issue 3(Suppl)

ISSN: 2167-0846 JPAR an open access journa

June 15-16, 2016 	Page 40

T Furuta et al., J Pai n Relief 2016, 5:3(Suppl) http://dx.doi.org/10.4172/2167-0846.C1.006

International Conference on

Fibromyalgia and Chronic Pain (June 15-16, 2016   Philadelphia,

Notes/Comments:

J Pain Relief 	Fibromyalgia 2016

ISSN: 2167-0846 JPAR an open access journa

June 15-16, 2016

Volume 5, Issue 3(Suppl)

Page 40