Effects of Controlled Atmosphere Storage and Temperature on Quality Attributes of Mango.

Zinabu Hailu, School of Chemical and Bio Engineering, Addis Ababa University, Ethiopia

Teshome Worku, School of Chemical and Bio Engineering, Addis Ababa University, Ethiopia

Z

ABSTRACT

Email: zinisheh@gmail.com

Phone: +2519-22098403

This is Zinabu Hailu from Ethiopia. I have B.Sc degree in Chemical Engineering and Msc in Chemical and Bio-Engineering/Food Engineering. I had also participated in short term training on “Laboratory management system (ISO17025)” by Ethiopian standard Agency which focused on quality handling of laboratory works. I have been working as Instructor and Research in Adigrat University, Ethiopia, in the Department of chemical engineering for more than four years. I have publications regard to Chemical engineering research works such as ‘Effects of Oxygen and Yeast on Quality Attributers Of Wine Produced From Ethiopian Beetroot” and “Preparation of Wine  From Cactus Fruit In  Adigrat Rural Areas”. Thus, I am interesting in participation of the conference

To date, only 3% of the total world production is being exported due to the perishable nature of the product, lack of proper storage and other postharvest related diseases. Post-harvest loss of mango in Ethiopia is estimated to be between 25-40% and would occur during handling, transportation and storage. CAS is agricultural technologies which monitor and adjust constantly the level of CO2 and O2 within storage to reduce the rate of physiological and biochemical changes, ethylene sensitivity and incidence of decay development of mangoes and also inhibit pathogen growth. Thus, this study was under taken to determine the optimum storage conditions of Ethiopian mango in order to reduce considerable postharvest mango losses. Accordingly, experiments were carried out at constant storage temperature of 7oC with different atmospheric conditions of A (10%CO2 + 6%O2+84%N2), B (8%CO2+ 6%O2+86%N2), C (5%CO2+5%O2+90%N2) and Control (0.03% CO2 + 21% O2+79%N2).  Besides, fruits also stored at different constant temperatures of 7 oC, 10 oC, 13oC and control (21-24 oC) respectively. The result obtained showed that effect of CAS on quality attributes (weight loss and firmness) of cv. Keitt mango significantly improved the storage life as compared to that stored at ambient atmosphere with list significant difference values being at 0.05=3.12 and 1.708 respectively. In addition, the effect of temperature on Total soluble solid, Titrable acidity, pH, weight loss, firmness and skin color was found to be significant in prolonging the storage life of the fruit with List significant difference values of 0.05(0.317, 6.58, 3.46, 4.148, 6.50 and 2.41).The study revealed that the fruit stored at 7oC under 10%CO2 +6%O2storage condition, as confirmed by sensory analysis, was given better preference to extend cv. Keitt mango up to 6weeks. On the other hand, the fruits stored at 13oC with no CAS conditions showed better storage time of up to 21 days without chilling injury effect. Finally, it can be conclude that application of CAS technology can significantly enhance the quality and storage life thereby minimizing post-harvest losses of mango fruit.

1. Desta H, (2005). Export potential of Ethiopia processed fruit and vegetables, export promotion department of English, P., S. Jaffee and J.J. Okello.2006.

2. Joosten F (2007). Development Strategy for Export Oriented Horticulture in Ethiopia. http://library.wur.nl/way/bestanden/clc/1891396.pdf

3. Doreyappy-Gowda, I.N.D and A.G. Huddar, 2001, Studies on ripening changes in mango (Mangiferaindica L.) fruits. J. Food Sci. Tec. Mysore, 38: 135- 137.

4. FAO, 2010: Technical guidelines on tropical fruit tree management in Ethiopia; Giuseppe De  Bac Project GCP/ETH/073/ITA

5. ILRI, 2011. Fruits -A synthesis of IPMS value-chain development experiences. ILRI, Nairobi.