# Nitrogen ion implantation effect on the physical properties of copper oxide thin films on glass substrate

Azadeh Jafari1, Hamed Vosoughikia2

1Plasma Physics Research Center, Science and Research Branch, Islamic Azad University, Tehran, Iran

2Department of Engineering, Sanandaj Branch, Islamic Azad University, Sanandaj, Iran

Abstract

One of the method for engineering the structural properties of a metal oxide to modify its optical and electrical properties, is nitrogen doping. In this context nitrogen doping in copper oxide is an important research topic because of its potential for overcoming the disadvantage of copper oxide its high resistance. In this paper effect of nitrogen ion implantation on copper oxide thin film deposited on glass substrate by DC magnetron sputtering has been studied.  In order to investigate the effect of the nitrogen ion implantation on copper oxide thin film, the crystallographic structure of the samples was obtained using X-ray diffraction method. Atomic force microscopy and scanning electron microscopy were used for surface morphology investigation and UV-VIS spectrophotometer was used for optical properties. The XRD patterns showed Cu2O:N formation with an orthorhombic structure in implanted  sample. The SEM images showed some significant change in the surface morphology after nitrogen ion implantation in such a way that some interconnected holes created on the surface. According to the AFM images, the roughness of the samples after implantation is decreased and changed due to the ballistic effect of the ions implanted. The study on optical properties showed that nitrogen ion implantation promoted the delocalization of charge carriers as a result, the optical band gap decreased. Another result of nitrogen ion implantation was reducing resistivity of samples which the I-V characteristics was performed with keithley2361system. The results provide a useful experimental instance of N doped copper oxide synthesis and would promote the research and applications of Cu2O:N devices such as photovoltaic material systems.

# Keyword: Nitrogen ion implantation, copper oxide thin films, XRD, SEM, AFM