# Direct integration of bio-gasoline production into current petrorefinery

Aron Deneyer, Researcher, Centre for Surface Chemistry and Catalysis (COK), KU Leuven, Belgium

Bert Lagrain, Industrial research manager, Centre for Surface Chemistry and Catalysis (COK), KU Leuven, Belgium

Michiel Dusselier, Professor, Centre for Surface Chemistry and Catalysis (COK), KU Leuven, Belgium

Bert F. Sels, Full Professor, Centre for Surface Chemistry and Catalysis (COK), KU Leuven, Belgium

### Abstract

Sugars, which are the main constituents of biomass, play an important role in the proposed transition towards renewables, but their full potential for the chemical industry is still underexploited. They have a unique carbon skeleton composed of 5 to 6 carbon atoms that are all connected in a linear way. Apart from the presence of oxygen atoms, these carbon chains are identical to the ones found in fossil light naphtha. This means that, providing a procedure for the selective removal of oxygen atoms, sugars potentially have an identical field of application as light naphtha. In this contribution, the direct hydrodeoxygenation of carbohydrate streams into light naphtha is presented. A one-pot biphasic (water:alkane) catalytic system is used, yielding liquid C5-C6 hydrocarbons directly from (ligno)cellulosic feedstocks. The catalytic reaction proceeds at medium temperature and under H2 pressure in the presence of an acidic as well as a redox catalyst. Some essential insights concerning the catalysts, the substrate as well as the organic solvent of this liquid phase cellulose-to-naphtha (LPCtoN) technology are discussed. Besides, a realistic process flow diagram, with LPCtoN as central technology, is presented. First, an efficient reductive catalytic fractionation (RCF), starting from woody resources, is demonstrated to produce a soluble lignin oil and a solid carbohydrate pulp. Second, the LPCtoN strategy, converting the carbohydrate pulp into naphtha, is fully integrated into current petrorefinery processes. The key to success for this integration is the use of fossil light naphtha streams as organic phase, resulting in a direct enrichment with bio-naphtha. This bio-enriched stream can immediately be used as substrate in a conventional hydro-isomerisation unit, after which a valuable (bio-)isomerate with a high octane number is obtained. This stream is extremely valuable for gasoline blends, especially taking into account the EU imposed directives of 10% green carbon in fuels by 2020.

### Image

Figure 1: Schematic representation of the valorisation of second generation biomass into phenolics (lignin oil) and bio-enriched gasoline.

Recent Publications

- Deneyer A, Peeters E, Renders T, Van den Bosch S, Van Oeckel N, Ennaert T, Szarvas T, Korányi TI, Dusselier M, Sels BF (2018) Direct upstream integration of biogasoline production into current light straight run naphtha petrorefinery processes. Nature Energy DOI: 10.1038/s41560-018-0245-6.

- Deneyer A, Renders T, Van Aelst J, Van den Bosch S, Gabriëls D, Sels BF (2015) Alkane production from biomass: chemo-, bio- and integrated catalytic approaches. Curr. Opin. Chem. Biol. 29:40-48.

- Deneyer A, Ennaert T, Sels BF (2018) Straightforward sustainability assessment of sugar-derived molecules from first-generation biomass. Curr. Opin. Green Sustain. Chem. 10:11-20.

- Op de Beeck B, Dusselier M, Geboers J, Holsbeek J, Morré E, Oswald S, Giebeler L, Sels BF (2015) Direct catalytic conversion of cellulose to liquid straight-chain alkanes. Energy Environ. Sci. 8:230-240.

- Deneyer A, Ennaert T, Cavents G, Dijkmans J, Vanneste J, Courtin CM, Dusselier M, Sels BF (2016) Compositional and structural feedstock requirements of a liquid phase cellulose-to-naphtha process in a carbon- and hydrogen-neutral biorefinery context. Green Chem. 18:5594-5606.

- Deneyer A, Tlatli S, Dusselier M, Sels BF (2018) Branching-first: synthesizing C-C skeletal branched biobased chemicals from sugars. ACS Sustainable Chem. Eng. 6:7940-7950.

Biography

Aron Deneyer obtained his MSc in Bioscience Engineering (Catalytic Technology) at KU Leuven in 2014. During his master thesis, he explored the potential of hierarchical zeolites as catalyst in the valorisation of terpenes. As an IWT-funded fellow (2015-2018), under the guidance of Prof. Bert Sels, he investigated in his PhD the catalytic biorefining of sugars towards molecules with low and high functionality. As a post-doctoral researcher, he is currently occupied with the synthesis of tailor-made zeolites.

Email: aron.deneyer@kuleuven.be

Notes/Comments:

Synthesizing short drop-in alkanes from (abundantly available second generation) sugar resources is a valuable valorisation route in the short term, considering the structural resemblance of the carbon backbone between sugar building blocks and naphtha.