Pretreatment of lignocellulosic biomass using Fenton, ozone and peroxone process: Impact on enhancement of reducing sugar concentration and biogas production

Sameena N. Malika,b*, Krishna Madhu, Atul. N. Vaidyab

aDepartment of Energy Science & Engineering, Indian Institute of Technology, Bombay, Maharashtra, India.

bCSIR – National Environmental& Engineering Research Institute, Nehru Marg, Nagpur, Maharashtra, India.

*Corresponding Author.  Email: naazmalik17@gmail.com

The efficacies of various advanced oxidation processes like fenton, ozone and peroxone on a lignocellulosic biomass like Yard waste was studied based on the enhancement effect of pretreatments on the TRS (Total Reducing Sugar) concentration. The best pretreatment on the basis of TRS concentration was ozone pretreatment. The enhanced TRS concentration was 2251 mg/l for ozone pretreated biomass whereas for pure yard waste without any pretreatment was 552.8 mg/l. The optimized conditions obtained for the ozone pretreatment were 1.9 g/ hr of ozone dosage at pH 3 for a duration of 15 minutes. Fenton and peroxone pretreatments also has indicated a positive effect on TRS concentration enhancement. BMP tests had also shown a positive effect for pretreated lignocellulosic biomass with highest methane content of 60% for ozone pretreatment as compared to 535, 42% and 45% for peroxone, fenton and control.

Reference

- Adepu Kiran Kumar, S. S. (2017). cent updates on different methods of pretreatment of lignocellulosic feedstocks: a review. Bioresources and Biopreocessing.

- Bajpai, P. (2016). Pretreatment of Lignocellulosic Biomass for Biofuel Production. SpringerBriefs in Green Chemistry for Sustainability,, 7-12.

- Dawn M. Kato, N. E. (2014). Pretreatment of lignocellulosic biomass using Fenton chemistry. Bioresource Technology, 273-278.

- Devendra Prasad Maurya, A. S. (2015). An overview of key pretreatment processes for biological conversion of lignocellulosic biomass to bioethanol. 3 Biotech, Springer, 597- 609.

- Furkan H. Isikgor, C. R. (2015). Lignocellulosic Biomass: A Sustainable Platform for Production of Bio-Based Chemicals and Polymers. Polymer Chemistry.

- Jitendra Kumar Saini, R. S. (2015). Lignocellulosic agriculture wastes as biomass feedstocks for second-generation bioethanol production: concepts and recent developments. Springer, Biotech, 337-353.

Professional Biography

Sameena Naaz Malik is pursuing PhD in Energy Science & Engineering Department at Indian Institute of Technology Bombay, India. She has completed her bachelor degree in Chemical engineering from Nagpur University, India. She has published 8 International papers in SCI journals. She is working in the area of wastewater treatment along with biofuel production.

Presenting Author Detail

Full Name: Sameena Naaz Malik

Contact number: 7020204189
Twitter account: Nil
Linked In account:naaz_1709@yahoo.in

Category: Oral presentation