# Response Surface Method Optimization of Rice Straw-Liquefaction using Crude Glycerol for Rigid Polyurethane Foam Application

Rosal Jane G. Ruda1, Kriztine M. Icalina, Arnold A. Lubguban, Cañaveral

Mindanao State University – Iligan Institute of Technology Iligan City, Philippines

### Abstract

Statement of the Problem: Polyurethane is a versatile class of polymer produced from the condensation polymerization of isocyanates and polyol, a hydroxyl-rich compound. Its application can be found in adhesives, sealants, coatings, flexible and rigid foams. However, polyol is traditionally sourced from petroleum raw materials. With increasing environmental and sustainability problems, lignocellulosic biomass is a potential alternative source due to its abundance, biodegradability and its hydroxyl component. In the Philippines, 11.3 M tons of rice straw is produced annually which when burned, produces air pollutants such as carbon dioxide, nitrogen oxide and sulfur dioxide. Rice straw can be liquefied using an atmospheric liquefaction process with the use of a catalyst. Researchers reported that varying liquefaction parameters can significantly alter the properties of a polyol3. The purpose of the study is to produce rice straw-based polyol with a low acid number, high OH number, low viscosity and high liquefaction ratio which is suitable for rigid foam applications. Methodology & Theoretical Orientation: A one-pot liquefaction process was used to liquefy rice straw with the use of sulfuric acid as catalyst and crude glycerol as liquefaction solvent. Response surface methodology was used to optimize four factors: acid loading, biomass loading, reaction time and reaction temperature based on four responses: acid number, OH number, liquefaction ratio, and viscosity.  Findings: Statistical analysis showed that all four factors have a significant effect on polyol properties. Increasing the acid loading was shown to significantly increase residual acid while higher reaction times lead to a decrease in liquefaction efficiency. Polyols with optimum properties were produced at a reaction time of 180-300 minutes, acid loading of 1-2%, reaction temperature of 170-180°C, and a biomass loading of 10-15%.

### Image

Figure 1. Reaction pathway of Cellulose Solvolysis by Acidified Glycerol4

Recent Publications

- Anwar, Z., Gulfraz, M., and Irshad, M. (2014). Agro-industrial lignocellulosic biomass a key to unlock the future bio-energy: A brief review. Journal of Radiation Research and Applied Sciences, 1-11.

- Brand, S., Kim, J. (2014). Liquefaction of major lignocellulosic constituents in supercritical ethanol. Energy, 1-11.

- Hu, S., Wan, C., and Li, Y. (2012). Production and characterization of biopolyols and polyurethane foams from crude glycerol based liquefaction of soybean straw.

- Kosmela, P., Hejna, A., Formela, K., Haponiuk, J., and Piszczyk, L. (2016). Biopolyols obtained via crude glycerol-based liquefaction. of cellulose: their structural, rheological and thermal characterization. Cellulose. DOI 10.1007/s10570-016-1034-7

- Purwanto, E., Sutrismo, J., Apriliana, R., and Horax, K.M. (2015). The optimization of biopolyol synthesis from liquefaction of rice straw using response surface method, Journal of Chemical and Pharmaceutical Research. 7(9S):46-53.

- Yang, F., Hanna, M.A., and Sun, R. (2012). Value-added uses for crude glycerol--a byproduct of biodiesel production. Biotechnology for Fuels. DOI: 10.1186/1754-6834-5-13

Biography

Rosal Jane G. Ruda is a licensed chemical engineer, currently taking up Master of Science in Materials Science and Engineering at Mindanao State University – Iligan Institute of Technology. She is also currently working as a University Research Associate under Dr. Arnold A. Lubguban working in polymer research. She graduated with honors in her undergraduate program. She was also given recognition for graduating under the RA 2687 Scholarship Program under the Department of Science of Technology.

Email: rosaljaneruda@gmail.com