Abstract Template

Title: Identification of new alleles in salinity tolerant rice local cultivars through phenotypic and genotypic screening

M.K. Papan, K. Venkatesan, I.T. Paulsen, L. Li, B. Raguse, J. Cooper. E. Chow

1

Macquarie university, Sydney, Australia

2

CSIRO, Sydney, Australia

Keywords: Gold nanoparticle chemiresistors, Antibacterial susceptibility diagnostics, Bacteria Metabolites, Healthcare diagnostics

- Abstract Body (up to 250 words)

The cornerstone of antimicrobial resistance management is appropriate use of antibiotic. Generally, the first and second clinical question for antimicrobial therapy are 1) whether patient needs antimicrobial therapy, and 2) If infection is present what antimicrobial is appropriate. Basically, current testing methods for selecting an effective antibiotic can be too slow (36–72h). Therefore, a technology for rapid susceptibility testing is required to replace culture-based methods and help clinicians to prescribe narrow-spectrum agents and defining antibiotics susceptibility profile.

Most of the commercially rapid methods for identifying antimicrobial resistance are genotypic methods. Genotypic methods provide information for predicting antimicrobial resistance not antimicrobial susceptibility.  It informs physician what not to prescribe rather than clear therapeutic option. Phenotypic antimicrobial susceptibility testing is still the ‘’gold Standard’’. This method is slow and require large No of viable organisms e.g. (broth dilution and disk diffusion techniques).

Chemiresistor sensors are amongst new technologies being investigated to meet present needs for diagnosing diseases. Potential advantages of the technology include its amenability to point of care diagnostics, inexpensive components and rapid response times we present work on utilizing gold nanoparticle chemiresistors for providing antibiotic susceptibility profile. The detection principle is based on the distinct metabolic differences associated with species specific bacterial growth.

Bacteria take in nutrients and release metabolites as they grow. The small molecular weight chemicals partition into the thiol functionalized gold nanoparticle (AuNP)film. In steady state, gold nanoparticles sit a distance ‘L’ apart. As chemicals partition into and out of the gold nanoparticle film the distance between gold nanoparticles changes by ‘L’ which cause the resistance of the AuNP film to change.

Four different bacteria including gram positive and gram negative and more importantly different strains of same pathogen which have different antibiotic susceptibility pattern has been tested with the sensor array. Also, as proof of concept, the sensor is capable enough to provide Minimum Inhibitory Concentration (MIC) results for different pathogens. Limit of detection for different inoculum size has also been measured.

To the best of our knowledge this is the first documentation of using AuNP chemiresistors for determining antibiotic susceptibility profile. The results suggest that detection may be possible in less than 24 h which would be a significant reduction in time compared to the 2 to 5 days it can take to identify infections using conventional culture methods. AuNP chemiresistors represent an approach to achieving rapid bacterial detection, identification and growth monitoring with inexpensive technology that is amenable to the development of point of care diagnostics or bioreactor monitoring. In future studies, this chemiresistor could be applied to detect infection directly from liquid clinical samples by detecting the metabolomic biomarkers produced by a host in response to infection.

IV.	Biography:

M.K. Papan is Phd student and Commonwealth Scientific and Industrial Research Organisation (CSIRO) in Sydney, Australia

(phone: 0061452115017; e-mail: (Mohammadkazem.papan.csiro.au, Mohammadkazem.papan.hdr.mq.edu.au).

K. Venkatesan, is Associate Professor at Macquarie university, Sydney, Australia e-mail: koushik.venkatesan@mq.edu.au

I.T. Paulsen is Professor at Macquarie university, Sydney, Australia.

L. Li is postdoctoral fellowship at Macquarie university, Sydney, Australia.

B. Raguse, J. Cooper and E. Chow are scientists at CSIRO, Sydney, Australia.

    - Presenting author details

Full Name: Mohammadkazem Papan

Contact number: 0061452115017

Twitter account:

Linked In account:

Session name/ number:0061452115015

Category: (Oral presentation/ Poster presentation): Ora; presentation

Passport Number: J34168022