# Innovative biosensor array for key analyte monitoring in human sweat

Daniele Zappi 1, Gabriele Varani 2, Maria Teresa Giardi 1,2.

1 Istituto di Cristallografia, CNR Area Della Ricerca di Roma, 00015 Monterotondo Scalo Rome, Italy

2 Biosensor Srl, Via Degli Olmetti 44, 00060 Formello Rome, Italy

### Abstract (300 word limit)

Statement of the Problem: Stress markers monitoring has been proved to be useful in order to better understand the overall physiological state of an individual. Conventional techniques for monitoring such markers require long and complex analyses on blood samples, at the same time putting the patient in a state of discomfort due to the need of repeated blood samplings in a short amount of time. Thus, the attention has moved to less invasive monitoring procedures based on fluids excreted by the human bodies during normal biological activity.

Methodology & Theoretical Orientation: Sweat has been proven to contain various biomarkers which can be well-correlated with the overall physiological state of the individual. Furthermore, sweat is normally excreted during normal life activities as well during physical exercise. Since sweat quantities are overall small, a system has been designed to capture and convey it towards stretchable screen-printed electrode, produced using renewable green materials, ad-hoc modified with enzymes suitable for the detection and quantification of target analytes such as glucose, lactic acid, uric acid and dopamine.

Findings: Through amperometric measurements, amounts of target analytes in sweat have been quantified. The measurements have been performed firstly on simulated sweat media, then on real human sweat samples. Finally, the system has been applied to volunteers, to monitor target analytes released in sweat during physical exercise.

Conclusion & Significance: The proposed sensing array has proven useful for detection, quantification and monitoring of analytes related to physiological stress released in sweat. Possible applications of the system can be supposed for monitoring and increasing performances of sportsmen and monitoring health status of workers in dangerous or critical workplaces.

### Image

Recent Publications (minimum 5)

- R. Bloomer, A. Goldfarb, L. Wideman, M. McKenzie, L.A. Consitt, Effects of acute aerobic and anaerobic exercise on blood markers of oxidative stress, J. Strength Cond. Res. (2005). https://doi.org/10.1519/14823.1.

- J. Moyer, D. Wilson, I. Finkelshtein, B. Wong, R. Potts, Correlation Between Sweat Glucose and Blood Glucose in Subjects with Diabetes, Diabetes Technol. Ther. 14 (2012) 398–402. https://doi.org/10.1089/dia.2011.0262.

- K. Enomoto, Y. Jingushi, H. Kudo, Sweat Lactic Acid Monitoring System for Assessment of Exercise Intensity, ECS Trans. 75 (2017) 61. https://doi.org/10.1149/07552.0061ecst.

- S. Farajikhah, P.C. Innis, B. Paull, G.G. Wallace, A.R. Harris, Facile Development of a Fiber-Based Electrode for Highly Selective and Sensitive Detection of Dopamine, ACS Sens. 4 (2019) 2599–2604. https://doi.org/10.1021/acssensors.9b01583.

- M.T. Giardi, D. Zappi, M. Turemis, G. Varani, F. Lo Celso, G. Barone, P. Calandra, A. Antonacci, V. Scognamiglio, Quantum dots functionalised artificial peptides bioinspired to the D1 protein from the Photosystem II of Chlamydomonas reinhardtii for endocrine disruptor optosensing, Talanta. (2020) 121854. https://doi.org/10.1016/j.talanta.2020.121854.

- EUCALIVA (EUCAlyptus LIgnin VAlorisation for Advanced Materials and Carbon Fibres), https://eucaliva.eu/

- S.A. Garimella, A. Senouci, K. Kim, Monitoring Fatigue in Construction Workers Using Wearable Sensors, (2020) 86–94. https://doi.org/10.1061/9780784482872.010.

Biography (150 word limit)

Dr. Daniele Zappi achieved Master’s Degree cum Laude in Analytical Chemistry in 2016 and obtained a PhD degree in Chemistry in 2019, both at University “La Sapienza” of Rome. Currently, he is a researcher at IC – CNR. His research focus is on the development of electrochemical and optical sensors and their optimization for analysis on complex matrixes.

Email: daniele.zappi@ic.cnr.it

Dr. Gabriele Varani is a chemical engineer with experience in the field of fluid-dynamics and mass transport applied to chemical and biological process. He received his master degree (2014) and the Ph.D. in chemical engineering (2018) at “La Sapienza” University of Rome. At present he is working at Biosensor s.r.l. His research focus is on the development of microfluidics devices, electrochemical sensors, software development and 3D printing

Email: g.varani@biosensor.it

Dr. Maria Teresa Giardi is associated researcher at the National Council of Research (CNR) in Rome, Italy. Her background is in organic-industrial chemistry and she has extensive experience in biochemistry; development of signal transduction systems for optical and electrochemical biosensors; protein stabilization and utilization in biosensor design. She has worked in various research institutions and research and development companies in both Europe and the USA. She is a supervisor or coordinator of a number of national and international projects in the field of biosensors: European Agency projects Frame V, VI, VII; European Space Agency projects involving space flights to ISS.

Email: mt.giardi@biosensor.it