Ultra-low noise organic based devices to record bioelectrical signals in non-excitable cell populations: Applications in anticancer drug screening platforms

Ana L. Mestre1,3, Rute .C. Felix1,2, Maria C. R. Medeiros3, Deborah M. Power2 and Henrique Leonel Gomes3,

1Universidade do Algarve, Faculdade de Ciências e Tecnologia, 8005-139 Faro, Portugal

2Centro de Ciências do Mar, Universidade do Algarve, 8005-139 Faro, Portugal.

3Instituto de Telecomunicações, Departamento de Engenharia Electrotécnica e Computadores, Universidade de Coimbra, 3030-290 Coimbra, Portugal.

In contrast with the extensive research on devices that explore neuron-neuron communication, the electronic technology to measure long-range communication in non-electrogenic (or non-nervous) cells is still in its infancy. Non-electrogenic cells generate weak signals (a few micro-volts) with frequencies below 1 Hz. Designing low noise devices in a millihertz frequency range is extremely challenging due to the intrinsic thermal and 1/f type noise generated by the sensing electrode. Our group has pioneered research in this area [1]–[3] and recently developed devices with extremely low electrical noise[4]. These novel devices can detect the faint electrical fluctuations generated by the cooperative activity of non-electrogenic cells. The capacity of devices to measure non-electrogenic cells was achieved by fabricating them using an inkjet-printed conducting polymer, the poly(3,4-ethylenedioxythiophene) polystyrene sulfonate (PEDOT:PSS). This soft and micro-fibrillar material enhances the effective area of the electrical double-layers formed between the electrolyte and the sensing surface. Furthermore, the PEDOT: PSS device is flexible, soft and biocompatible.

As an example of application of our devices and methods, we demonstrated that we can monitored in real-time the synchronization of cancer cells as they migrate and invade a cell free surrounding region [5]. Since cell bioelectrical activity is a result of a cell synchronization process, our devices also provide important information about cell-cell connectivity.

In this contribution, we present and discuss the sensing electrode specifications, namely the geometry, the design as well as the type of electroactive materials that can minimize the intrinsic electrical noise in the millihertz frequency range. The ultra-high sensitivity of our sensing surfaces was evaluated by recording electrophysiological signals from non-electrogenic or brain tumour cells in vitro. In a bandwidth of 12.5 Hz a noise level of 0.1-0.2 V r.m.s was achieved.

In addition to the device specifications, we also discusses how low noise polymer based devices can be used to measure in real time and over extended periods of time (weeks) the effect of drugs on the bioelectrical activity of cells. The goal is to establish the basis for a drug-screening platform relying on extracellular measurements of the cell bioelectrical activity. A variety of cells populations have been evaluated, including glioma cells and fibroblast type of cells.

Acknowledgements – The authors gratefully acknowledge support from the Portuguese Foundation for Science and Technology through the UIDB/50008/2020 (Instituto de Telecomunicações, IT) and UID/Multi/04326/2020 (Centro de Ciências do Mar, CCMar). RCF was funded by FCT, under the “Norma Transitória” -DL57/2016/CP1361/ project CT0020 and AM was funded by an FCT PhD grant SFRH/BD/148688/2019.

References

[1]	M. C. R. Medeiros et al., “An electrical method to measure low-frequency collective and synchronized cell activity using extracellular electrodes,” Sens. Bio-Sensing Res., vol. 10, pp. 1–8, Sep. 2016.

[2]	A. L. G. Mestre et al., “Ultrasensitive gold micro-structured electrodes enabling the detection of extra-cellular long-lasting potentials in astrocytes populations,” no. October, pp. 1–11, 2017.

[3]	P. R. F. Rocha et al., “Extracellular electrical recording of pH-triggered bursts in C6 glioma cell populations,” Sci. Adv., vol. 2, no. 12, p. e1600516, Dec. 2016.

[4]	P. M. C. Inácio et al., “Ultra-low noise PEDOT:PSS electrodes on bacterial cellulose: A sensor to access bioelectrical signals in non-electrogenic cells,” Org. Electron., vol. 85, no. June, 2020.

[5]	S. Asgarifar et al., “Extracellular electrophysiological based sensor to monitor cancer cells cooperative migration and cell-cell connections,” Biosens. Bioelectron., vol. 145, no. August, p. 111708, Dec. 2019.