## Strategy of screening and optimization of process parameters using experimental design: application to a layer of aluminum 1050A by vibratory grinding treatment

## , K.HAMOUDA2, M.H.E.AMROU1 ,V.IVANOV3, A. P BABICHEV3

1, 2, University of Sciences and Technology Houari Boumediene, BP 32, El-Alia, 16111, Bab-Ezzouar, Algiers, Algeria.

3,Don State Technical University, Rostov on Don, Russia

Abstract

A full factorial design (FFD) 23 was used as screening technique to verify the influence of the three factors on the response variable (thickness of the oxide layer: Tol) by vibratory grinding treatment on an aluminum alloy 1050A.  Factors studied are the amplitude oscillation of the working chamber (A = 2.0 - 5.0 mm), treatment Time (t = 30-60 min), treatment frequency (f = 25-33,3 Hz). The values of these parameters were selected from a preliminary study on formation of the oxide layer at the material surface. As a result of FFD evaluation, the main influent factors are the amplitude of oscillations (A) and treatment Time (t). The fitted quadratic regression model, including these parameters was developed using the central composite design (CCD) as the response surface methodology. Analysis of variance (ANOVA) was validated with a confidence level > 90%. The final model is established on the basis of additional statistical analysis using other statistical criteria (Mallow’s Cp statistic), the convenient results obtained, ensuring a satisfactory quadratic regression model, including all main parameters and their quadratic terms. Optimal conditions were determined using an analytic method, stationary point of model was determined, and the values of Hessian matrix determinant showed that the response (thickness of the surface layer: Tol) agrees a maximum solution corresponding to optimal conditions, A = 2,32 mm and t = 35,12 min, at which the predictive value of the response is Tol = 4,99μm.

Keywords: central composite design, full factorial design, optimization, aluminum 1050A, vibratory grinding treatment.

References

[1] A.P. Babichev, I.A. Babichev, Fundamentals of vibration technology: Rostov on Don: Publishing Center DGTU, 1999 – 621 p.

[2] L. Eriksson, E. Johansson , N. Kettaneh-Wold, C. Wikström , S. Wold. Design of experiments. Principles and Applications, third revised and enlarged edition. MKS Umetrics AB., Umeå, Sweden. 1999-2008.

[3] , G.E.P. Box, J.S. Hunter. Multi-factor experimental design for exploring response surfaces, Annals of Mathematical Statistics 28 (1957) 195–241.

[4] M.H.Kalavathy, I. Regupathi, M.G. Pillai, L.R. Miranda. Modelling, analysis and optimization of adsorption parameters for H3PO4 activated rubber wood sawdust using response surface methodology (RSM). Colloids Surf. B 70 (2009) 35–45.

[5] A. Azargohar, A.K. Dalai. Production of activated carbon from luscar char: experimental and modeling studies.  Micropor. Mesopor. Mater. 85 (2005) 219–225.

[6] D.C.Montgomery. Design and Analysis of Experiments, fifth ed. John Wiley, New York. 2001.

[7] Sérgio L.C. Ferreira, Walter N.L. dos Santos, Cristina M. Quintella, Benicio B. Neto, Juan M. Bosque-Sendra. Doehlert matrix: a chemometric tool for analytical chemistry—review.. Talanta 63 (2004) 1061–1067.

[8]  A.P. Donald. Optimization theory with applications, Dover Publications, New York. 1986.

[9]  P.L. Dawson, R. Martinez-Dawson .Using response surface analysis to optimize the quality of ultrapasteurized liquid whole egg.Poult. Sci. 77 (1998) 468–474.

[10] V. Caqueret, S. Bostyn, C. Porte, H. Fauduet, Optimization of the operating conditions for the removal of alcoholic insoluble compounds contained in sugar beet vinasse, Chem. Eng. J. 145 (2008) 203–210.

[11] D. Wu, J. Zhou, Y. Li, Effect of the sulfidation process on the mechanical properties of a CoMoP/Al2O3 hydrotreating catalyst, Chem. Eng. Sci. 64 (2009) 198–206.