Flowcytometry: Principle, Interpretation of result and its role in solid tumors

Principle: Flowcytometer is equipment which has become an integral part for the diagnosis of hematolymphoid malignancies. The word ‘flow’ means to pass, ‘cyto’ means cell & ‘metry’ means measurement. The flowcytometry is the passage of cells in single file (line or row) in front of a laser to be detected, counted and sorted. Cells labeled with fluorochrome are striked by laser to emit light (fluorescence) at varying wavelengths. Fluorescence (photon) is filtered and collected to convert the result into a digitalized (numerical) value. The reading is done by specialized software. Cells of interest flow through a liquid stream. The speed of flow of cell is higher than the speed of fluid carrying the cell (sheath fluid). This results into arrangement of cells in a single line (single file). This mechanism is called Hydrodynamic focusing. Up to 50000 cells per second can be measured, but the normal throughput is 1000 – 10000 cells/ sec (1). Detectors of photons are called photomultiplier tube. The detector placed in the line of light beam measures Forward Scattering (FSC) [size] and that of placed perpendicular to the light stream measures Side Scattering (SSC) [granularity, nuclear structure]. For all practical purposes cells falling in the range of 3-20 µ in diameter can be analyzed (2).  Identification of rare cells at frequencies as low as 0.0001% has been reported (3).

Information about physical and chemical structure of cells is gathered, which is used in diagnosis of diseases. Samples used are bone marrow, blood, body fluid and tissue. For tissue dissociation of cell are required to separate it in to single cell. Equipment for Tissue dissociation is available commercially.  For example GgentleMACS™ OctoDissociator

Getting numerical values: Detectors collect photon, gets converted into electrical energy (current) to give a digitized value through “Analog to Digital Converter”. Simply the detector is left to emit the photons. The time taken by the detector to emit all the photons gives a numerical value. This can be plotted on a graph. Common soft wares in used are Caluja, CellQuest, Flowjo, FCS Express (FCS: Fluorescence activated cell sorting), etc.

Uses of Flowcytometer: Immunophenotyping to differentiate Acute leukemia into  Lymphoid/ myeloid/biphenotypic/bilineage. In Lymphoid malignancies, whether it is T-cell or B-cell. It has numerous uses in research work like DNA cell cycle/tumor ploidy, Chromatin structure, Total protein, lipids, surface charge, Enzyme activity, DNA synthesis, DNA degradation, Gene expression etc.

Comparison with Immunohistochemistry (IHC):

Cytometry (IHC):

- Localization of antigen is possible: Nuclear / cytoplasmic,

- Poor enumeration of cell subtypes

- Simultaneous measurements of cells not possible

FlowCytometry (FCM):

- Cannot tell you where the antigen is, whether nuclear or cytoplasmic?

- Can analyze many cells in a short time

- Can look at numerous parameters at once

To summarize, in present era, FCM has become a necessity of a laboratory, which has been dealing with hematolymphoid malignancy. No therapy of a case of leukemia can be even imagined without having facility of Flowcytometry. As all the chemotherapeutic agents are having serious side effect. So without getting correct diagnosis chemotherapy can’t be started.

1. Mittag A, Tarnok A; Perspective in cytometry; Chapter 1; Book- Advanced Optical Flow 
Cytometry: Methods and Disease Diagnoses, edited by Valery V. Tuchin; Wiley Publishers, 
2011

2. Gilbraith DW; Isolation and flowcytometric characterization of plant protoplast; Book, 
Methods in Cell Biology, Vol 33; Edited by Zbigniew Darzynkiewicz and Harry A. Crissman; 
Acdemic Press, INC; P. 528 (1990).

3. https://www.thermofisher.com

Contributed by:

Dr Amar Ranjan, Assistant Professor, DR.BRAmbedkar - Institute Rotary Cancer Hospital, All India Institute of Medical Sciences, New Delhi – 110029

E MAIL: dr.amarranjan@rediffmail.com