Production of agar from algal sea weed from coastal sites of Karachi.

Noureen Arif1, Rehmat Khan2, Kausar Khan3 Afsheen Arif3

1.Bahria University, Karachi

2.University of Management and Technology

3.University of Karachi.

Abstract:

Pakistan is the country rich in many natural resources out of these, seaweeds are one of the them. The luxuriant natural occurring algal resources can be utilized to produce compounds which are been imported in huge amount. This raw material along with proper extraction and purification serve the purpose in a crude manner. The study is based to produce agar and agarose compounds extracted from local red algal species like Gracilaria corticate, Gelidium pusilum.  The coastal region has over sixty species of seaweed out of which red algal species are abundant [1]. The species Gracilaria corticate was seasonal in from 1989, 1990, 2000, 2012. It highest biomass was recorded in the month of April [2]. The biomass ranges from 5 to 60 g.m2. The study aims to optimize the procedure for the production of agar. The samples should be clean on site of collection to detached shells, sand and other algae. Dry the sample as soon as possible to prevent deterioration. Weigh the sample and calculate the impurities and then dry it [3]. After drying the samples will be bleached with lime and alkali treatment. The polysaccharides in Gracilaria species are agaroids and low in gel strength, it is treated with aqueous sodium hydroxide solution to remove the sulfate and the strength of gel increases by 10 folds which is better than Gelidium species. The one kg sample will be treated with 20% NaOH for 3 days and washed properly to remove traces and dried again, this product alkali treated seaweed is exported from Taiwan and Chile [4]. This process is summarized in the flow chart.

The small scale production can be further taken up to large scale production and farming of the red algal, to strengthen blue economy in the country.

Washed and Cleaned Seaweed

Extraction

Filtration

Gelation

Gelidium Species:

Heat with very dilute acid for one hour to improve water penetration

Gracilaria Species

Treat with alkali and wash

Bleaching Dialysis

Thawing

Freezing

Bleaching Dialysis

Syneresis

Drying

Milling

AGAR

References:

Qari, R. and Siddiqui S. A., 1993. Biochemical composition and yield of agar from the Gracilaria corticata of Karachi. Mar. Res., 2: 77-81.

Armisen, R. & Galatas, F. 1987. Production, properties and uses of agar. pp. 1-57.

Stirk, W.A. & van Staden, J. 2000. Removal of heavy metals from solution using dried brown seaweed material. Botanica Marina, 43: 467-473.

Kapraun, D.F. 1999. Red algal polysaccharide industry: economics and research status at the turn of the century. Hydrobiologia, 398/399: 7-14.