Emissions of СО2, СН4 and N2O from drainage of organic forest soils due to conversion of forest land in Russia.

The land use, land-use change and forestry (LULUCF) sector has a significant impact on gas exchange between the atmosphere and terrestrial ecosystems.

The main contribution to the emission of greenhouse gases in the LULUCF sector in the 20th and early 21st centuries is caused by deforestation, which mainly occurs in tropical regions where agriculture is actively developing, and large areas of tropical forests are burned for agricultural purposes. The fastest forest growth rates have been observed since the mid-20th century, the annual CO2 emissions from deforestation in the tropics and subtropics of South America and Asia are estimated at 1.3 billion tons from year-1 (Richter, Kias).

In the temperate and boreal zones, these processes practically do not occur, the volume of greenhouse gas (GHG) emissions is not significant, forest lands can be reduced for the construction of infrastructure facilities (roads, power lines, gas pipelines, oil pipelines, etc.), but the estimation of emissions from deforestation is envisaged in the international reporting on the UN Framework Convention on Climate Change (UNFCCC) and the Kyoto Protocol (Protocol), and the countries that sign them annually submit their results in the National Greenhouse Gas Inventories.

In order to improve the accuracy of GHG emission calculations for deforestation, presented in the national greenhouse gas inventory of the Russian Federation, we carried out an assessment of CO2, CH4 and N2O emissions during the dehumidification of organic soils during the transfer of forestland to settlement lands.

To estimate the area of ​​organic soils, the average share of the land area with excessive moisture from the total forest area in the average for the Russian Federation was used, which is 21.7% (Sirin, 2001). To estimate the total area of ​​organic soils of forest lands transferred to settlements, annual deforestation areas were multiplied by the proportion of organic soils in forests.

Considering that drainage of excessively moistened soils during the construction of linear infrastructure facilities is extremely rare due to the high cost of this activity, in Russia, as a rule, dehumidification is not applied. Building norms provide for cheaper options: bypassing such lands, building road and / or railroad tracks on piles, overpasses, immersing gas and oil pipelines to the bottom of water bodies. In extreme cases, it is assumed that the soil will be filled with excessively moistened soils throughout the road and / or railroad tracks. At the same time, the original peat soil is not seized, but is pushed back artificially with bulk soil to the sides. Based on the study of this material, it was accepted conservative assumption that in 10% of cases, organogenic soils during deforestation are drained. This value was used in the calculations.

Annual emissions of CO2, CH4 and N2O are calculated using the following formulas and conversion factors presented in the methodology (Wetlands Supplement, 2013):

CО2_organic = A_drained * EF * 44/12                                                                                        (1)

Where,

СО2_organic - СО2 emissions from drained soils, tons of СО2;

A_drained - area of ​​drained soils, ha;

EF is the СО2 emission factor from the drained soils, tons С ha-1 year-1 (the mean value between the default coefficients for poor and rich organic soils in the boreal forest is 0.71 tons С ha-1 year-1).

N2O_organic = A_drained* EFN_N2O * 44/28                                                                           (2)

Where,

N2O_organic - emissions of N2O from drained soils, kg N2O;

A_drained - area of ​​drained soils, ha;

EFN_ N2O is the emission factor of N2O from drained soils, kg N- N2O ha-1 year-1 (value of 1.71 kg of N- N2O ha-1 year-1).

Estimation of methane emissions from drained organic soils is carry out according to formula (3).

CH4_organic = A_drained * (1-Fracditch) * EF_land + A_drained * F_racditch * EF_ditch        (3)

Where,

CH4_organic - emissions of methane, kg CH4;

A_drained - area of ​​drained soils, ha;

Frac_ditch - the share of the total area under the drainage channels has no dimension;

EF_land - emission factor for plots not occupied by drainage channels, kg CH4 ha-1 year-1;

EF_ditch - emission factor for drainage ditches, kg CH4 ha-1 year-1.

The following coefficients were used for forests in the boreal zone (average between the default parameters for poor and nutrient-rich organic soils):

Frac_ditch = 0.025 CH4 kg ha-1 year-1;

EF_land = 4.5 CH4 kg ha-1 year-1.

EF_ditch = 217 CH4 kg ha-1 year-1.

The area of ​​drained organic soils and the corresponding N2O and CH4 emissions are present in Table 1.

The total area of ​​drained land for the period from 1990-2016 years is estimated at 153 thousand hectares or an average of 10 thousand hectares per year. The total annual emissions of СО2 for the period under review amounted to 433 thousand tons or an average of 28.9 thousand tons per year. CH4 emissions amounted to 1.4 thousand tons or 0.09 thousand tons. N2O total emission is estimated at 0.45 thousand tons, an average of 0.03 thousand tons per year.

Table 1.

The results of calculations of carbon losses and CH4 and N2O emissions from organic soils during transfer of forest land to settlements.

|  | 1990 | 1995 | 2000 | 2005 | 2006 | 2007 | 2008 | 2009 | 2010 | 2011 | 2012 | 2013 | 2014 | 2015 | 2016 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Area of drained organic soils, thousand hectares | 1,1 | 5,0 | 7,7 | 9,6 | 9,9 | 10,3 | 10,8 | 11,2 | 11,5 | 11,8 | 12,2 | 12,5 | 12,9 | 13,1 | 13,4 |
| СO2, thousand tons | 3,3 | 14,3 | 21,6 | 27,1 | 28,2 | 29,3 | 30,4 | 31,5 | 32,6 | 33,4 | 34,8 | 35,6 | 36,3 | 37,0 | 37,8 |
| CH4, thousand tons | 0,0 | 0,1 | 0,1 | 0,1 | 0,1 | 0,1 | 0,1 | 0,1 | 0,1 | 0,1 | 0,1 | 0,1 | 0,1 | 0,1 | 0,1 |
| N2O, thousand tons | 0,003 | 0,015 | 0,022 | 0,028 | 0,029 | 0,030 | 0,031 | 0,033 | 0,034 | 0,035 | 0,036 | 0,037 | 0,038 | 0,038 | 0,039 |