Title: Impacts of Land-use Changes and Urbanization on Coastal Groundwater Resources of Lagos, SW-Nigeria: Integrated Hydrogeochemical and GIS-based Assessments

TIJANI, M.N.1,  ADEKOYA, A.E.2,  FASHAE, O.A.3,  TIJANI, S.A.2  and  ALADEJANA, J.A.1

1Department of Geology, University of Ibadan, Ibadan – Nigeria (E-mail: tmoshood@gmail.com).

2Dept. of Agric. Extension and Rural Development, University of Ibadan, Ibadan - Nigeria.

3Department of Geography, University of Ibadan, Ibadan – Nigeria.

Keywords: Land-use changes, Urbanization, Coastal groundwater, Hydrogeochemistry, Saline Intrusion, Water Quality, Lagos.

- Abstract

Water and food security are essential to human survival and well-being vis-a-vis sustainable resources management and attainment of the SDGs. This study employed hydrogeochemical and GIS-based assessments of impacts of land use and urbanization on groundwater with emphasis on the resilience and security of coastal water supply in Lagos metropolis, SW-Nigeria.

For the assessment of land-use changes and associated impacts on community livelihood, Spot 5 satellite imagery of 1984, 2002 and 2017 of Lagos area alongside with Landsat ETM+ and OLI were used to generate the land use / land cover (LULC) changes in ArcGIS 10.3 Software. For the hydrogeochemical quality assessment, seventy (70) water samples were collected from different household water points (wells and boreholes) within Lagos. In-situ physicochemical parameters (pH, Temperature, TDS  and EC) were measured in the field followed by laboratory analyses of major cations and anions.

GIS-based assessment revealed a significant increase in the built-up areas from 447.3km2 in 1984 to 951km2 in 2017 representing 85% increase with corresponding loss of about 5% of the forested coastal wetlands. This is a clear indication of rapid urbanization in the last three decades with attendant impacts on coastal water quality and sustainability. Hydrochemical assessment revealed elevated electrical conductivity (EC) of >1,000μS/cm alongside with elevated Na (51-230mg/l), Cl (300-980mg/l) and SO4 (36-88mg/l) concentrations in shallow wells and boreholes tapping the upper unconfined aquifer. This is a clear fingerprint of groundwater vulnerability to saline intrusion and contamination.

Therefore, in the face of reality of impacts of anthropogenic land-use and climate-induced changes, there is the need for a clear governance structure to regulate the rate of groundwater abstraction and indiscriminate groundwater development to ensure sustainable groundwater resources management and attainment of SDG-6 and avoidance of water supply crisis. Such crisis can be avoided, if appropriate knowledge-based choices, planning / management options are employed; failure of which will likely be very costly.

### Image

- Reference

- Bilgehan, N., and, Ali, B., (2010). Groundwater quality mapping in urban area using GIS. Environ. Monitoring. Assessment. 160:215–227.

- Howard, K.W.F., and Mullings, E., (1996). Hydrochemical analysis of groundwater-flow and saline incursion in the Clarendon Basin, Jamaica. Groundwater 34(5):801–810.

### Biography

Prof. M.N. Tijani obtained his doctorate degree from University of Muenster, Germany in 1997 and had undertaken post-doctoral studies at the University of Hiroshima in Japan between 2003 and 2005. As a certified geoscientist, he has published more 65 papers in reputed journals and attended several international conferences around the World.

### Presenting authordetails

Full Name: Moshood Niyi TIJANI

Contact number:  +2348023252339

Session name/ number:

Category: Oral presentation

Passport Number: A07575368.