Title: Inter-Comparison of Energy Consumption Patterns in Bull's Trench and Zig Zag Brick Kilns of Nepal

ASHISH RAI1, AMIT CHAUDHARY1, SAGAR ADHIKARI2, SUJAN SHRESTHA2,3, KUNDAN LAL SHRESTHA1, BHISHMA PANDIT2, SIVA PRAVEEN PUPPALA2

1Department of Environmental Science and Engineering, Kathmandu University, Dhulikhel, Kavre

2International Center for Integrated Mountain Development (ICIMOD), G.P.O. Box 3226, Kathmandu, Nepal

3Department of Environmental Science, Baylor University, Waco, Texas, USA

Keywords:  Brick kiln, fuel, fixed chimney bull’s trench kilns, induced draught zigzag kilns, specific energy consumption, air quality.

- Abstract Body

The brick industry plays a significant role in Nepal's energy consumption and contributes largely to our national economy. Large quantities of coal and other combustible fuels are used as major energy sources in firing bricks. In this study, we summarize results for seven kilns which include four straight line Fixed Chimney Bull’s Trench Kilns (FCBTKs) and three Induced Draught Zigzag (IDZZ) kilns.  This study particularly focuses on measurement of fuel feeding rate, temperature distribution, and flue gas parameters for several cycle of fuel feeding to quantify energy consumption and performance. Specific energy consumption (SEC) is one of the widely accepted methods of studying energy performance of brick kilns in terms of fuel consumption and total brick production.

We found the SEC values of 1.15±0.29 MJ/kg of brick fired for FCBTKs and 0.84±0.23 MJ/kg of brick fired for IDZZ technology. The SEC for IDZZ kiln is lower than FCBTK with the difference of 0.31 MJ/kg. At the daily production of 35,000 bricks, the potential for coal saving by adoption of IDZZ kiln over FCBTK is around 21700 MJ.  This will save around 800kg of coal per day. Complete energy auditing of four FCBTKs and three IDZZ kilns was performed during this campaign. Flue gas losses and surface losses were found to be 33.33±4.03% and 27.27±5.10% of total energy provided for FCBTK. Similarly, flue gas and surface losses for IDZZ kilns were found to be 18.98±3.46% and 18.13±0.90%, respectively, of total energy input. This measurement shows that the flue gas losses and surface losses were among the major losses for

FCBTKs and were significantly higher when compared to IDZZ kilns. Hence, our results suggest that adoption of IDZZ over FCBTK technology paves the way for energy conservation, air quality improvement, and on-site worker health safety.

- Image:

- Reference:

1. Maithel, S., Lalchandani, D., Malhotra, G., Bhanware, P., Uma, R., Ragavan, S., & Athalye, V. (2012). Brick Kilns Performance Assessment A Roadmap for Cleaner Brick Production in India. Shakti Sustainable Energy Foundation and Climate Works Foundation, New Delhi.

2. PACE Nepal (2012). Baseline study of selected sector industries to assess the potentials for more efficient use of energy, Nepal Energy Efficiency Programme (NEEP)/GIZ.

3. Pokhrel, R., & Lee, H. (2014). Integrated Environment Impact Assessment of Brick Kiln using Environmental Performance Scores. Asian Journal of Atmospheric Environment, 8(1), 15-24.

- Biography:

Mr. Ashish Rai has completed his undergraduate degree (B.tech. in Environmental Engineering) at the age of 23 years from Kathmandu University, Nepal. He is currently working as a consultant engineer in Green Decision consultancy. He has presented his bachelor thesis work in more than two conferences.

- Presenting author details

Full Name: Ashish Rai

Contact number: +977-9842429274
Twitter account: https://twitter.com/ashishkoyu18
Linked In account: https://www.linkedin.com/in/ashish-rai-17b6a8135/

Session name/ number:
Category: Poster presentation

Passport Number: 10804864