Assessment to heavy metals in road side dust along the Abu Dhabi-Alin National highway, UAE

### Yousef Nazzal1, , Ahmed A. Al-Taani2, Fares M. Howari1

### 1College of Natural and Health Sciences Zayed University P O Box 144534 Abu Dhabi, UAE, Yousef.nazzal@zu.ac.ae, fares.howari@zu.ac.ae,

### 2Department of Earth and Environmental Sciences, Faculty of Science, Yarmouk University, Irbid-21163, Jordan, taaniun@hotmail.com

Abstract

Concentrations of key heavy metals (Pb, Zn, Cd, Ni, Cr, Mn, As and Hg) have been investigated in roadside dust collected from Abu Dhabi and Al-Ain National Highway in UAE. Heavy metal levels varied widely from 227.9-2765, 19-1540, 37.4-398.6, 20.1-123, 0.3-0.7, 0.1-0.9, 0.1-0.7 and 0.1-0.5 mg/kg for Mn, Cr, Zn, Pb, Cd, Hg, Ni and As, respectively. The spatial distribution pattern showed that different sources of roadway metal emissions dominate at specific locations. Peaks in heavy metal concentrations were frequently observed in locations with high traffic volume, road intersections and junctions, gas and bus stations. Decreased levels of heavy metals were measured in locations with low traffic loadings and in close proximity to farm and forest areas. In addition to traffic related heavy metals, emissions from fossil fuel and industries remain of significant importance. Natural sources, through regular dust storms, are important contributors to the observed metal levels. Enrichment factor indicated that As and Ni were entirely originated from crustal sources. Cd, Zn, Pb, and Mn were moderately enriched and are probably derived from mixed sources (Traffic flows, Gas stations, construction activities, agricultural activities ……etc). Road dust were significantly enriched in Cr and Hg indicating their dominant anthropogenic origin and strong contribution from non-crustal sources. The average geo-accumulation index values suggest that road dust are uncontaminated with Mn, Ni, and As, uncontaminated to moderately contaminated with Zn and Pb, and moderately contaminated with Cr, Cd and Hg. The contamination factors indicate very highly contaminated road dust with Cr and Hg, considerably contaminated with Cd, moderately contaminated with Mn, Zn and Pb, and slightly contaminated with Ni and As. Results of ecological risk assessment revealed that all heavy metals in road dust pose low risk local to ecosystems, except for Cd and Hg, which constitute  potentially considerable risk and high risk, respectively.

Keywords: Heavy metals, road side dust, contamination factor, geo-accumulation index, ecological risk assessment, UAE