Coconut Waste as Bioresource for Sustainable Energy Development:

Quantifying the Waste, Energy Content and Emissions

1Amoah Y. D, 2Ofori-Agyemang C, 3Obeng G.Y,  4Mensah, E.

1,2,4 Department of Agricultural and Biosystems Engineering, College of Engineering, Kwame Nkrumah University of Science and Technology, Kumasi, Ghana.

3 Department of Mechanical Engineering, College of Engineering, Kwame Nkrumah University of Science and Technology, Kumasi, Ghana.

### Abstract (300 word limit)

After the edible portions of coconut fruit are consumed, the husks and shells are thrown away or burned raw as domestic fuel, particularly for the energy-poor. Improper disposal and burning of coconut waste result in environmental pollution. This study sought to quantify the waste, energy content, PM2.5 and CO emissions from burning of coconut waste for sustainable bio-char energy production in Kumasi, Ghana. Cross-sectional data were gathered from primary sources by administering questionnaires to 110 coconut sellers. Coconut wholesale points, dump sites and landfill sites in Kumasi were also surveyed. Furthermore, 50 fresh coconuts (25 local and 25 hybrid varieties) were randomly sampled, weighed, dehusked and the residue was used in an experiment. The study revealed that several tonnes of coconut waste are generated per month out of which 91% is dumped. Of the total mass of fresh coconut, 61-65% is generated as waste (shells 14-15%, husks 47-50%). Energy content of raw uncharred coconut waste was 9-16 MJ/kg and charred coconut waste was 17-21 MJ/kg. These values compared favorably to wood (12-16 MJ/kg) at 20% moisture content. However, PM2.5 and CO were higher compared to WHO 24-hr AQG values at 25 °C, 1 atm. Higher concentrations of PM2.5 and CO emissions can adversely affect human health and the atmosphere. Hence to effectively utilize the total mass and energy content of coconut waste for sustainable energy, there is the need to biochar under a controlled environment to increase energy content, reduce emissions and maintain optimal energy production conditions.

Image

Recent Publications (minimum 5)

- Ayamga, E. A., Kemausor, F. and Addo, A. (2015). Technical analysis of crop residue biomass energy in an agricultural region of Ghana.  Journal of Resources Conservation and Recycling 98. p. 51-60.

- Bradley, W. L., Poel, J.  and Huang, H. (2006). Cocos nucifera: An Abundant Renewable Source of Energy. Proceedings of the International Conference on Renewable Energy for Developing Countries. p. 1-5. Available on http://www.udc.edu/docs/cere/Cocos%20Nucifera-paper.pdf on March 8, 2016.

- Duku, M. H., Gu, S., and Hagan, E. B. (2011). A comprehensive review of biomass resources and biofuels potential in Ghana. Renewable and Sustainable Energy (15), p. 404–415.

- Ganiron, J. T. U. (2013). Sustainable Management of Waste Coconut Shells as Aggregates in Concrete Mixture. Journal of Engineering Science and Technology. 6(5), 7-14.

- Ofori-Agyeman, C. (2016). Assessment of Coconut Waste Management: A Case Study Kumasi Metropolitan Assembly. Unpublished undergraduate thesis, Kwame Nkrumah University of Science and Technology, Ghana.

Biography (150 word limit)

George Yaw Obeng is an Associate Professor of Mechanical Engineering and former Director of Technology Consultancy Centre, Kwame Nkrumah University of Science and Technology (KNUST), Ghana. He is Barrett Global Fellow and Senior Sustainability Fellow, Julie Ann Wrigley Global Institute of Sustainability of Arizona State University, USA where he served as a visiting faculty from fall 2016 to spring 2017. He is also a Fellow of The Energy Center, KNUST. He has been a faculty of KNUST for over 23 years. He teaches quantitative methods, spatial statistics, renewable energy for rural applications and sustainable energy applications for developing world and has been practically involved in the development and study of agro processing equipment. His primary research areas include renewables (solar, bioenergy), environment and sustainable engineering design. He holds a PhD in Development Studies with focus on energy access and development, MSc in Appropriate. Technology and Development (major in Renewable Energy Technologies) from Flensburg University, Germany, and BSc (Hons) in Agricultural Engineering from KNUST

Email: gyobeng.coe@knust.edu.gh

Notes/Comments: