# Manufacturing and enhanced performance of metallic lattice structures by additive manufacturing

Lai-Chang Zhang, Professor, Edith Cowan University, Australia

### Abstract

Lattice structures (Periodic cellular materials) are a type of architectures constructed by an array of spatial periodic unit cells with edges and faces. Although lattice structures exhibit a range of superior properties, such as low elastic modulus, high specific stiffness and strength, large surface area, a large number of internal pores, and relatively low-stress level, the manufacturing by traditional technologies and the optimization of properties of lattice structured are limited because of poor control over the morphological characteristics of the pore network.  The recent advances in metal additive manufacturing (AM) technologies have enlarged the design and manufacturing possibilities for lattice structures, which provides lattice structures with promising solutions for a variety of applications. This work presents the recent progress of additive manufacturing and enhanced mechanical and catalytic performance of lattice structures. The selective laser melting (SLM) produced and electron beam melting (EBM)-produced beta-type biomedical titanium lattice structures, as potential load-bearing implants, demonstrate excellent mechanical properties in terms of high super-elasticity, low Young’s modulus and high compression strength, high energy absorption and fatigue properties. In addition, SLM-produced porous Fe-based metallic glass composite has a superior overall catalytic ability with high reaction rate constant and low activation energy than other catalysts. The reported reusability and overall catalytic ability in SLM-produced porous Fe-based metallic glass matrix composite hold the promise to design new generation catalyst approaching practical application and high economic value.

### Recent Publications

- Liu YJ, Li SJ, Wang HL, Hou WT, Hao YL, Yang R, Sercombe TB, Zhang LC* (2016) Microstructure, defects and mechanical behavior of beta-type titanium porous structures manufactured by electron beam melting and selective laser melting. Acta Materialia 113:56-67.

- Liu YJ, Wang HL, Li SJ, Wang SG, Wang WJ, Hou WT, Hao YL, Yang R, Zhang LC* (2017) Compressive and fatigue behavior of beta-type titanium porous structures fabricated by electron beam melting. Acta Materialia 126: 58-66.

- Liu YJ, Li SJ, Zhang LC*, Hao YL, Sercombe TB (2018) Early plastic deformation behaviour and energy absorption in porous β-type biomedical titanium produced by selective laser melting. Scripta Materialia 153:99-103.

- Zhang LC*, Jia Z, Lyu F, Liang SX, Lu J. (2019) A review of catalytic performance of metallic glasses in wastewater treatment: recent progress and prospects. Progress in Materials Science 105:100576.

- Liang SX, Wang XQ, Zhang WC, Liu YJ, Wang WM, Zhang LC* (2020) Selective laser melting manufactured porous Fe-based metallic glass matrix composite with remarkable catalytic activity and reusability. Applied Materials Today 19:100543.

- Liu YJ, Ren DC, Li SJ, Wang H, Zhang LC*, Sercombe TB (2020) Enhanced fatigue characteristics of a topology-optimized porous titanium structure produced by selective laser melting. Additive Manufacturing 32:101060.

Biography

Lai-Chang Zhang is a Professor of Materials Engineering and the Program Leader–Mechanical Engineering in the School of Engineering at Edith Cowan University (Perth, Australia). After awarded his PhD in Materials Science and Engineering at the Institute of Metal Research, Chinese Academy of Sciences, Prof. Zhang held several positions at The University of Western Australian, University of Wollongong, IFW Dresden and Technical University of Darmstadt. His research interests include metal additive manufacturing, light-weight alloys, nanocrystalline materials and metallic glasses, and nanomaterials for water treatment. He has published about 280 referred journal papers with an H-index of 60 and 10000+ citations and 22 ESI Highly Cited Papers. He also served as Editors or Editorial Board Members for several journals, e.g. Advanced Engineering Materials, Metals, Frontiers in Materials, Materials Science and Technology, etc.

Email: l.zhang@ecu.edu.au; lczhangimr@gmail.com

Notes/Comments: