# Synthesis and Characterization of Mg0.5Zn0.5Fe2O4 coated with Polymer (Polyvinyl Alcohol) Nanocomposites via ultrasound assisted emulsion liquid phase

Mujahid Mustaqeem, Department of Chemistry, The Islamia University of Bahawalpur, Bahawalpur-63100, Pakistan

Tawfik A.Saleh, Department of chemistry, King Fahd University of Petroleum & Minerals (KFUPM), Dhahran, Saudi Arabia

Aziz ur Rehman, Department of Chemistry, The Islamia University of Bahawalpur, Bahawalpur-63100, Pakistan

### Abstract

Nanocomposites of Mg0.5Zn0.5Fe2O4 –Polyvinyl alcohol were successfully synthesized using two steps: the nanoparticles of Mg0.5Zn0.5Fe2O4 were synthesized via micro-emulsion and then embedded into polyvinyl alcohol (PVA) matrix by ultrasound assisted emulsion liquid phase method. This approach can be solving problems in the dispersion and stabilization of Mg0.5Zn0.5Fe2O4 nanoparticles in the PVA matrix. The result showed that the prepared nanoparticles of Mg0.5Zn0.5Fe2O4 diffuse homogeneously in a polyvinyl alcohol matrix, maintaining the particle shape and size of the Mg0.5Zn0.5Fe2O4 nanoparticles. It was found that the aggregation of Mg0.5Zn0.5Fe2O4 nanoparticles could be reduced under ultrasonic irradiation. High resolution transmission electron microscopy (HRTEM) images reveal the presence of spherically shaped nanoparticles in the polyvinyl alcohol chains with an average size of 22nm. The XRD patterns indicate that the crystalline structure of composite is single phase cubic and the crystallite size decreases gradually from 44 to 22 nm by increasing concentration (0.1-0.5g) of nanoparticles with PVA. The interaction between the PVA and nanoparticles in nanocomposites samples were confirmed by Fourier-transform infrared spectroscopy via the shifting of bands revealed from Fourier-transform infrared spectra. The dielectric properties are found to be higher for Mg0.5Zn0.5Fe2O4 -reinforced blend systems than the pure blend system due to polarization exerted by  the incorporation of nanoparticles.Dielectric constant and loss factor shows the decreases trend with varying concentrations of nanoparticles with constant polymer concentration. This occurred due to the grain boundary effect which becomes dominant at low frequencies.

### Image

Flow chart for the synthesies of nanocomposites
Recent Publications

- Mujahid Mustaqeem, Khalid Mahmood, Tawfik A. Saleh, Aziz ur Rehman, Muhammad Ahmad, Zaheer Abbas Gilani, Muhammad Asif (2020), Synthesis of CuFe2–xErxO4 nanoparticles and their magnetic, structural and dielectric properties. Physica B: Condensed Matter 588: 412176.

- Warda Hassan, Sajida Noureen, Mujahid Mustaqeem, Tawfik A. Saleh, Shagufta Zafar (2020), Efficient adsorbent derived from Haloxylon recurvum plant for the adsorption of acid brown dye: Kinetics, isotherm and thermodynamic optimization. Surfaces and Interfaces 20: 100510.

- Mujahid Mustaqeem, Tawfik A. Saleh, Aziz ur Rehman, Muhammad Farooq Warsi, Azhar Mehmood, Adnan Sharif, Shahbaz Akther (2020), Synthesis of Zn0.8Co0.1Ni0.1Fe2O4 polyvinyl alcohol nanocomposites via ultrasound-assisted emulsion liquid phase. Arabian Journal of Chemistry. 13:3246-3254.

- Tawfik A. Saleh, Saddam A. AL-Hammadi, Ibrahim Munkaila Abdullahi, Mujahid Mustaqeem (2018), Synthesis of molybdenum cobalt nanocatalysts supported on carbon for hydrodesulfurization of liquid fuels. Journal of Molecular Liquids. 272:715-721.

- Z. A. Gilani, M. S. Shifa, A. D. Chandio, m. N. Usmani, H. M. N. Ul huda K. Asghar, S. Aslam , M. Khalid, A Perveen, J. Ur rehman, Mujahid Mustaqeem, M. A. Khan (2018), Thermogravimetric analysis, optical and dielectric properties of newly developed LiNi0.5PrxFe2−xO4 nanocrystalline ferrites. Digest Journal of Nanomaterials and Biostructures, 13:809-816.

Biography

Mujahid Mustaqeem has expertise in the synthesis and characterization of nanoparticles, nanocomposites, materials chemistry and analytical chemistry. He has experience in research, evaluation and teaching. He has expertise in micro-emulsion , solo-gel and co-precipitation methods for the synthesis of nanoparticles. He has strong expertise to utilize XRD, FTIR, Dielectric data to interpreted it.

Email: mustaqeem.mujahid@gmail.com