# Some features of zirconium isotopes separation by the method of laser assisted retardation of condensation

Konstantin Lyakhov1 and Heon-Ju Lee1,

1Institute for Nuclear Science and Technology, Applied Plasma Physics Laboratory, Department of Nuclear and Energy Engineering of Jeju National University, Ara-dong 1, Jeju, 63243, South Korea

### Abstract (300 word limit)

Commercial ores used for deriving zirconium for pressurized water reactor parts have a hafnium contents amounting to 1-2%, which along with Zr-91 deteriorates fuel utilization efficiency substantially(fuel cost saving of 7% would result provided only Zr-90 is left). Therefore, some cheap way to eradicate them is in required. Possibility of using purification method based on laser assisted retardation of condensation is investigated. In this method, proper choice of molecule for selective excitation of target isotope is crucial. Because of significant overlap photo-absorption spectrum of dicyclo-pentadienil zirconium dicyanate with CO2 laser emission spectrum along with high vapor pressure at relatively modest temperature, these molecules were selected. On the basis of transport model, developed by Jeff Eerkens for this method, enrichment factor and product cut evolution over gas flow core pressure and temperature are found. Due to lack of experimental data on dimer bond vibrational spectra, the latter have been derived by using Gaussian software. Nozzle wall profile, that can provide temperature and pressure, corresponding to the maximum of enrichment factor, has been calculated as well.

Recent Publications (minimum 5)

- Lyakhov,K.A., Lee, H.J.,“New experimental setup for zirconium isotopes separation by the Laser Assisted Retardation of Condensation Method”, KAPRA & KPS/DPP Conference, June(2013)

- Lyakhov,K.A., Lee, H.J., “Perspectives of Industrial Separation of Zirconium Isotopes by the Laser Assisted Retardation of Condensation Method”, Springer Proceedings in Energy, 99-104 (2015)

- Lyakhov,K.A., Lee, H.J.,“Optimal design of experimental setup for boron isotopes separation by the Laser Assisted Retardation of Condensation Method”, Journal of Nanoscience and Nanotechnology, 15,11, November 2015, 8502-8507(6).

- Lyakhov,K.A., Lee, H.J.,“ Some Features of Experimental Setup Design for Isotopes Separation by the Laser Assisted Retardation of Condensation Method”, Journal of Laser Applications,27,022008 (2015).

- Lyakhov,K.A., Lee, H.J.,“ Some Issues of Industrial Scale Silicon Isotopes Separation by the Laser Assisted Retarded Condensation Method (SILARC)”, Springer Proceedings in Energy, 153-160 (2016)

- Lyakhov,K.A., Lee, H.J., Pechen, A.N.,“ Some Features of Boron Isotopes Separation by the Laser-Assisted Retardation of Condensation Method in Multipass Irradiation Cell Implemented as a Resonator”, IEEE Journal of Quantum Electronics,52,12,1400208-1-8(2016)

- Lyakhov,K.A., Lee, H.J.,Pechen, A.N.,“ Some Issues of Industrial Scale Boron Isotopes Separation by the Laser Assisted Retarded Condensation(SILARC) Method”, Separation and Purification Technology, 176(2017) 402-411.

Biography (150 word limit)

I have earned my PhD in theoretical physics from J.W. Goethe University(Frankfurt) in 2008 at age of 29. The title of my thesis was “Baryon Stopping and Quark-Gluon Plasma Production at RHIC and LHC”. Over next ~1.5 years I was involved in oil reservoir engineering in “Petroleum Technologies”. In the beginning of 2010, I became a Researcher in Institute of Biochemical Physics of Russian Academy of Science. There, I was mainly occupied by polymers and fuel cells modelling. In late 2010, I’ve joined the Plasma Applications Laboratory, Nuclear and Energy Engineering Department of Jeju National University, as a Researcher, and from 2012 as a Research Professor. Currently, I am in involved in the projects related to laser(isotopes separation, quantum control) and plasma(Thomson scattering, MHD generator) applications. I have published 16 papers in SCI journals, and gave 27 talks in international conferences.

.

Email: lyakhov2000@yahoo.com

Notes/Comments: