# Structure and rheology studies of montmorillonites aqueous solutions

Mourad Gareche, PhD, Université M’Hamed Bougara Boumerdes (UMBB)-Algérie

Nadjet Azril, PhD, UMBB-Algérie

Leila Saoudi, PhD, UMBB-Algérie

### Abstract

Statement of the Problem: this paper studies the rheological behaviour of montmorillonites aqueous solutions for various mass concentrations. The several techniques: X-ray scattering to determine their composition and structure, the light scattering to determine the diffusion coefficient of the clay particles and their size and the rheometry to determine the viscoelastic moduli were used to investigate the structure formed by the bentonite suspensions. The rheological measurements show that the bentonite particles are structured to form a fractal network. Knowing the structure of these suspensions and characteristic time, on the one hand, using Piau model, we have derived its fractal dimension (D=1.74 ± 0.08) and on the other hand, using Chambon and Winter model, we tried to model their shear complex modulus G*(ω).

Keywords: vicoelasticity, rheometry, fractal, montmorillonite, structure

### Image

Figure 1: Variation of the complex shear modulus as a function of frequency For the bentonite suspension 5%.

Recent Publications

- Zaccarelli E (2007) Colloidal gels: equilibrium and non-equilibrium routes. J. Phys. Condens. Matter 19, 323101.

- Vermant  J, Solomon MJ (2005) Flow-induced structure in colloidal suspensions. J. Phys. Condens. Matter 17, R187.

- Tombácz E Szekeres M (2006) Surface charge heterogeneity of kaolinite in aqueous suspension in comparison with montmorillonite. Appl. Clay Sci. 34, 105–124.

- Helgeson ME, Moran SE, An HZ, Doyle PS (2012)  Mesoporous organohydrogels from thermogelling hotocrosslinkable nanoemulsions. Nat. Mater. 11, 344–352.

- Katz A, Min X, Steiner JC, Trusiak A, Alimova A, Gottlieb P K B (2013) Influence of cations on aggregation rates in mg-montmorillonite. Clay Clay Miner. 61 (1–2), 1–10.

- Le Pluart, T., Duchet, J., Sautereau, H., Halley, P., Gerard, J.F., 2004. Rheological properties of organoclay suspensions in epoxy network precursors. Appl. Clay Sci. 25, 207–219.

- Osuji CO, Kim C, Weitz DA (2008) Shear thickening and scaling of the elastic modulus in a fractal colloidal system with attractive interactions. Phys. Rev. E 77 (060402), 1–4.

Biography

Mourad GARECHE received his PhD (2013) and HDR (2016) from University M’Hamed Bougara of Boumerdes (UMBB)-Algeria. Now, he is Lecturer and Reseacher in laboratory engineering of hydrocarbons-UMBB. His research areas include rheology, complex fluids, structure-fluid interaction, viscoelasticity, porous media, enhanced oil recovery and nanofluids. His research results include over 30 Engineer and Master graduates, 2 photocopied lessons, over 50 refereed journal and conference papers and supervision 4 theses.

Email: gareche.m@gmail.com

Notes/Comments: