# Materials Research 2022

Nanostructured thermoelectric perovskite materials

Dr.Megha.U1,2and  Hind.N1,2

1Department of Physics, DGMMES Mampad College, P.O Mampad, Malappuram,Kerala,India-676542. 2Department of Physics, Govt.Arts  and Science College, Meenchanda Kozhikode,Kerala,India.

# Keywords: Thermoelectric, perovskites, nanostructuring, ZT value, power generation

- Abstract Body

The power generation from renewable energy resources has got vast attention as the conventional resources are being depleted. In internal combustion engines only 25% of energy is usable for vehicle mobility and accessories, approximately 40% of the fuel energy is wasted as exhaust gas, 30% is dissipated in the engine coolant and 5% is lost as radiation and friction. Here comes the importance of the thermoelectric materials which can harvest waste heat from sunlight, combustion of fossil fuels, nuclear decay, chemical reactions, vehicles etc and convert it into electricity. Statistical survey shows that more than 60% of world’s energy loss is in the form of heat. The high performance thermoelectric materials can easily convert this heat into usable electrical energy. The thermoelectric system is an eco-friendly energy conversion technology with the advantages of high reliability, small size, feasibility in a wide temperature range and has no pollutants. The efficiency of these materials is defined in terms of figure of merit, ZT, which determines the thermoelectric performance. ZT= S2 σT/ κ, where S is the Seebeck Coefficient, σ is the electrical conductivity, κ is the thermal conductivity and T is the absolute temperature. Inorder to get good performance, the value of ZT should be high. This can be achieved by increasing both the Seebeck coefficient and electrical conductivity and reducing the thermalconductivity. Nano structuring of thermoelectric materials enhance the performance of material by quantum confinement effects and phonon scattering. The development of thermoelectric material will be a milestone for the modern technology. Perovskites are fascinating group of materials which shows the thermoelectric effect. The nanostructuring of these materials enhances the figure of merit of these materials by phonon scattering and quantum confinement effect.

# Image:

- Reference:

- Effect of Bi and Sr doping on morphological and magnetic properties of LaCo0.6Fe0.4O3 nanosized perovskites’. Megha U, George Varghese and Shijina K, Bulletin of Material Science 39 (2016) 125-131

- Nanosized LaCo0.6Fe0.4O3 perovskites synthesized by citrate-sol gelautocombustion method’. Megha U, Shijina K and George Varghese, Processing and Application of Ceramics 8 [2] (2014) 87-92

- Room temperature AC impedance and dielectric studies of Bi and Sr doped PrCo0.6Fe0.4O3 perovskites. ’. Megha U, George Varghese and Shijina KProcessing and Application of Ceramics 11(1):52-59

# Biography:

Dr.Megha.U  has completed his PhD at the age of 29 years from University of Calicut, Kerala,India and working as Assistant Professor in Department of Physics, DGMMES Mampad College,Malappuram, Kerala.She also recognized as Research Supervisor in Govt.Arts and science college  Meenchanda, Calicut, Kerala, India. She has 8 publications in reputed Journals mainly in Elsevier and a book chapter is accepted to publish in Intech open acess book publishers, U.K.

Dr.Megha.U

Full Name: Dr.Megha.U

09048029595

Linked In account: Megha unikoth

Session name/ number: Speaker

Oral presentation