# PREPARATION AND CHARACTERIZATION OF CELLULOSE ACETATE REINFORCED NANOCOMPOSITE FLUORESCENCE FILM FOR APPLICATION AS FILTER AND BIO-PACKAGING MATERIALS WITH ANTIBACTERIAL PROPERTIES

Azeh Yakubu1, Gabriel Ademola Olatunji2, Folahan Amoo Adekola2, David Olugbenga Adetitun3

*Corresponding Author`s e-mail: yakubuazeh@gmail.com or azehy@ibbu.edu.ng

1 Department of Chemistry, Ibrahim Badamasi Babangida University, Lapai, Niger State, Nigeria

2Department of Industrial Chemistry, University of Ilorin, Ilorin, Nigeria

3Department of Microbiology, University of Ilorin, Ilorin, Nigeria

Abstract

Room temperature cellulose acetate reinforced nanocomposite thin fluorescence films have been prepared using cellulose acetate, methylcellulose, cellulose and propylcellulose nanoparticles obtained via acid hydrolysis as re-enforcement at various loading through solution casting-evaporation method. Modulus increased with nanoparticle loading. Samples with maximum compatibility were cellulose nanoparticles (CNPs) loading of 40% with modulus of 113.290Mpa and Toluene diisocyanate (TDI) modified (TDI-CNPs) at 20% loading had 145.965Mpa. Others include cellulose acetate nanoparticles (CANPs) at 30% loading with 73.035Mpa; methylated cellulose nanoparticles suspension (MCNPs) with 5% loading had a modulus of 87.252Mpa, pure CA had 45 Mpa. The nanocomposite thin films were applied as filter for the removal of Bacillus; Enterococcus and Micrococcus sp. cells from crude bacteriocin. The nanocomposite films were re-usable after sterilization in autoclave at 102oC with recoverability of 95.9% (R2 = 0.959) based on of bacteriocin produced. Nanocomposite films showed antibacterial properties against clinical isolates; Pseudomonas 1, 2 and Proteus.

Keywords: Nanocomposite films, Filter, Nanoparticles, bacteriocin, antibacterial