### A new analytical method for the simultaneous extraction of compounds of different nature and some of their metabolites by liquid phase microextraction

Samira Dowlatshaha,b, Mohammad Sarajib, Rut Fernández Torresa, Miguel Ángel Bello Lópeza, María Ramos Payána,*

a Department of Analytical Chemistry, Faculty of Chemistry, University of Seville, c/Prof. García González s/n, 41012, Seville, Spain

bDepartment of Chemistry, Isfahan University of Technology, Isfahan 84156–83111, Iran

*corresponding author: ramospayan@us.es

The development of new procedures for the simultaneous extraction of compounds of different nature becomes especially important when analyzing biological samples [1,2]. In this work, a new miniaturized methodology based LPME was developed for simultaneous determination of five different families of analytes and two metabolites. The device was combined with a HPLC-UV system for the separation and determination of the model analytes in the samples. Optimum µLPME conditions (0.5 µL/min of donor phase at pH 3.5, 3 µL of NPOE as SLM, and 1 µL/min of acceptor phase at pH 11.5) allowed the efficient, accurate and repeatable determination of ten drugs (amoxicillin, sulfadiazine, sulfamerazine, tiamphenicol, ethyl 4-hydroxybenzoate, flumequine, propyl 4-hydroxybenzoate, 5-hydroxyl, 3- hydroxyl and diclofenac). The downscaled method exhibited more environmental and economic advantages and enjoyed enhancement of the portability of liquid phase microextraction systems. The method was successfully applied in urine samples with efficiencies over 80% for all analytes.

References

- Maria Ramos Payan; Santiago Maspoch; Andreu Llobera. An effective microfluidic based liquid-phase microextraction device (μLPME) for extraction of non-steroidal anti-inflammatory drugs from biological and environmental samples. Talanta 165, 496 - 501.

- Maria Ramos Payan; Santiago Maspoch; Andreu Llobera. A simple and fast Double-Flow microfluidic device based liquid-phase microextraction (DF-µLPME) for the determination of parabens in water samples. Talanta 165, 496-501