Lab-on-glass system based on thin film optoelectronic devices

and thick film microfluidics

Nicola Lovecchio1, Francesca Costantini2, Augusto Nascetti3, Alessio Buzzin1, Lorenzo Iannascoli3, Giampiero de Cesare1, Domenico Caputo1

1 Department of Information Engineering, Electronics and Telecommunications, Sapienza University of Rome, Rome, Italy

2 CREA-DC Research Centre for Plant Protection and Certification, Rome, Italy

3 School of Aerospace Engineering, Sapienza University of Rome, Italy

Keywords: Lab-on-chip, Thin film technology, Microfluidics, DNA amplification, Virus detection

- Abstract Body

Lab-on-Chip technology is gaining great interest due to the many possibilities that it offers in the fields of life sciences, from parallel analysis in genomics to point-of-care devices in medical diagnostics. At the beginning, Lab-on-Chip devices essentially consisted of a microfluidic network that miniaturized the analytical procedures leading to faster reaction kinetics and lower sample and reagents consumption. Recent devices integrate on a single substrate several functional modules, that allow all the functions of a human-scale test laboratory including transferring samples, drawing off a precise volume of a chemical product, reagent mixing, detection and quantification of biomolecules.

Within this framework, this work focuses on the development of a lab-on-glass system based on thin film optoelectronic devices and thick film microfluidics. Indeed we have integrated, on a single glass substrate, p-i-n amorphous silicon structures acting as photosensors and temperature sensors for for on-chip detection and temperature control and optical filters for selection of specified wavelength.

This glass substrate coupled with a microfluidic network made in polydimethylsiloxane (PDMS) or pressure sensitive adhesive layer, and connected to custom electronic boards, enables  applications in the field of mycotoxin detection and DNA amplification of bacteria and virus.

- Images:

Left:  Glass substrate integrating  amorphous silicon sensors and optical filters. Right: Example of PDMS based microfluidics.

IV.	Biography:

Domenico Caputo is Associate Professor with tenure at DIET at the University of Rome “La Sapienza”. His main research fields concerned the development of amorphous silicon photodetectors for detection of radiation from the UV to the near infrared range and of innovative electronic devices based on amorphous silicon. The present research interest is focused on the development of thin film photodetector and Lab-on-Chip system for DNA amplification and mycotoxin detection. He is referee of several scientific journals, author of more than 100 papers on international journals and principal investigator of national and international projects.

    - Presenting author details

Full Name: Domenico Caputo

Email id: Domenico.caputo@uniroma1.it

Contact number:

Linked In account:

Session name/ number: Lab-on-A-Chip Technology

Category: Oral presentation