# Displacive Transformations and Mechanism of Shape Reversibility in Shape Memory Alloys

Osman Adiguzel, Firat University, Department of Physics, Elazig, Turkey

### Abstract

A series of alloy systems take place in class of advanced smart materials with adaptive properties and   stimulus response to the external changes. Shape memory alloys take place in this group, due to the shape reversibility and capacity of responding to changes in the environment. These alloys exhibit a peculiar property called shape memory effect, which is characterized by the recoverability of two certain shapes at different temperatures. This phenomenon is initiated by cooling and deformation processes and performed thermally on heating and cooling. Therefore- this behavior can be called thermoelasticity. Strain energy is stored in the material with the plastic deformation and released on heating by recovering the original shape in bulk level. These alloys have dual characteristics called thermoelasticity and superelasticity, from viewpoint of memory behavior.  Two successive displacive transformations, thermal and stress induced martensitic transformations govern shape memory phenomena in crystallographic basis. Thermal induced transformation occurs occur with the cooperative movement of atoms by means of shear-like mechanism in <110>-type directions on {110} -type planes of austenite matrix, along with crystal twinning on cooling and ordered parent phase structures turn into twinned martensite structures, and twinned structures turn into the detwinned structures by stressing material in low temperature condition by means of stress induced transformation. Superelasticity is performed mechanically by stressing and releasing material at a constant temperature in parent phase region, and shape recovery is performed simultaneously upon releasing the applied stress. Superelasticity is performed in non-linear way; stressing and releasing paths are different in the stress-strain diagram, and hysteresis loop refers to energy dissipation. The elementary processes involved in such martensitic transformations are lattice invariant shear, lattice twinning and detwinning. It is well known that crystal twinning and detwinning reactions play a considerable role in shape memory effect and superelasticity.

Copper based ternary alloys exhibit shape memory in β-phase field. Lattice invariant shear and twinning is not uniform in these alloys, and cause to the formation of long period layered martensitic structures with lattice twinning on cooling.

In the present contribution, electron diffraction and x-ray diffraction studies performed on two copper based CuZnAl and CuAlMn alloys. Electron diffraction patterns and x-ray diffraction profiles exhibit super lattice reflection. Specimens of these alloys aged at room temperature, and a series of x-ray diffractions were taken duration aging. Reached results show that diffraction angles and peak intensities change with aging time. Especially, some of the successive peak pairs providing a special relation between Miller indices come close each other, and this result refers to the rearrangement of atoms in diffusive manner.

Keywords: Shape memory effect, martensitic transformation, thermoelasticity, superelasticity, lattice twinning, detwinning.

Recent Publications

- O. Adiguzel, Phase Transitions and Microstructural Processes in Shape Memory Alloys, Materials Science Forum Vol. 762 (2013) pp 483-486, (2013) Trans Tech Publications, Switzerland

- O. Adiguzel, Self-accommodating Nature of Martensite Formation in Shape Memory Alloys, Solid State Phenomena Vol. 213 (2014) pp 114-118, © (2014) Trans Tech Publications,

- O. Adiguzel, Nanoscale Aspects of Phase Transitions in Copper Based Shape Memory   Alloys, Nanotechnology in the Security Systems, NATO Science for Peace and Security Series C: Environmental Security 2015, pp. 131-134

- O. Adiguzel, Phase Transitions and Elementary Processes in Shape Memory Alloys, Advanced Materials Research Vol. 1101 (2015) pp 124-128, Trans Tech Publications, Switzerland

- O. Adiguzel, Nano-Scale Mechanisms in the Formation of Displacive Transitions in Shape Memory Alloys, Physics, Chemistry and Applications of Nanostructures - Proceedings of the International Conference Nonmeeting - 2013. Edited by Borisenko Victor E et al. Published by World Scientific Publishing Co. Pte. Ltd

- O. Adiguzel, Ageing dependence and martensite stabilization in copper based shape memory alloys, IOP Conf. Series: Materials Science and Engineering 60 (2014) 012001, Proceedings of International Symposium on Advanced Materials (ISAM2013), 23-27, September 2013, Islamabad, Pakistan

- O. Adiguzel, Thermoelasticity, Superelasticity and Nanoscale Aspects of Structural Transformations in Shape Memory Alloys. In: Struble L., Tebaldi G. (eds) Materials for Sustainable Infrastructure (2018), GeoMEast 2017.Sustainable Civil Infrastructures. Springer,

Biography

Dr Adiguzel graduated from Department of Physics, Ankara University, Turkey in 1974 and received PhD- degree from Dicle University, Diyarbakir-Turkey. He has studied at Surrey University, Guildford, UK, as a post-doctoral research scientist in 1986-1987, and studied on shape memory alloys. He worked as research assistant, 1975-80, at Dicle University and shifted to Firat University, Elazig, Turkey in 1980. He became professor in 1996, and he has already been working as professor. He published over 60 papers in international and national journals; He joined over 100 conferences and symposia in international and national level as participant, invited speaker or keynote speaker with contributions of oral or poster. He served the program chair or conference chair/co-chair in some of these activities. In particular, he joined in last seven years (2014 - 2020) over 80 conferences as Keynote Speaker and Conference Co-Chair organized by different companies.  He supervised 5 PhD- theses and 3 M.Sc.- theses.

Dr. Adiguzel served his directorate of Graduate School of Natural and Applied Sciences, Firat University, in 1999-2004. He received a certificate awarded to him and his experimental group in recognition of significant contribution of 2 patterns to the Powder Diffraction File – Release 2000. The ICDD (International Centre for Diffraction Data) also appreciates cooperation of his group and interest in Powder Diffraction File.

Email: oadiguzel@firat.edu.tr