Effects of Biocompatible Substrates on the Electrical Properties of Graphene

M. Simchia, M. Berahmana, E. Rezvanib, I. Mirzaeia, M. Amiria, A. Simchib, c, M. Fardmanesha*

a Department of Electrical Engineering, Sharif University of Technology, Tehran, 113658639, Iran

b Department of Materials Science and Technology, Sharif University of Technology, Tehran, 113658639, Iran

c Institute for Nanoscience and Nanotechnology, Sharif University of Technology, Tehran, 113658639, Iran

*fardmanesh@sharif.edu

### Keywords: Graphene; Sheet resistance; Biocompatible substrates

### Abstract

### Graphene is a single-atomic two-dimensional crystal of carbon atoms that has considerable properties due to its unique structure and physics with applications in different fields. Graphene has sensitive electrical properties due to its atomic-thin structure. Along with the substrate materials and their influence on the transport properties in graphene, design and fabrication of graphene-based devices for biomedical and biosensor applications are challenging. In this work, large-area high-quality graphene nanosheets were prepared by low pressure chemical vapor deposition using methane gas as carbon source on copper foil and transferred on the biocompatible substrates. Through deposition of titanium and gold contacts, current-voltage response of the transferred graphene on four biocompatible substrates including PDMS, SU-8, Nitrocellulose, and Kapton were experimentally determined.  The considerable effect of the substrate type on the electrical properties of graphene is shown. The sheet resistance of graphene is changed from 0.34 to 14.5 kῼ/sq depending on the substrate.
Recent Publications

- ] K. S. Novoselov et al, (2004) “Electric Field Effect in Atomically Thin Carbon Films”, Science, 306, 666.

- R. S. Edwards and K. S. Coleman, (2013) “Graphene synthesis: relationship to applications”, Nanoscale, 5, 38.

- F. Kiani, Z. Razzaghi, B. Ghadiani, M. Tamizifar, M.H. Mohmmadi, A. Simchi, (2017) “Self-limited growth of large-area monolayer Graphene films by low pressure chemical vapor deposition for Graphene-based field effect transistors”, Ceramics International, 43, 15010.

- W. Zhu, V. Perebeinos, M. Freitag, and P. Avouris, (2009) “Carrier scattering, mobilities, and electrostatic potential in monolayer, bilayer, and trilayer Graphene”, Phys. Rev. B, 80, 235402.

5. C. Tang, M. Li, L. J. Li, C. C. Chi, and J. C. Chen, (2011) “Characteristics of a sensitive micro-Hall probe fabricated on chemical vapor deposited Graphene over the temperature range from liquid-helium to room temperature” App. Phys. Lett., 99, 112107..