Ultra-Fast Heat Dissipating Aerogels Derived from Polyaniline Anchored Cellulose Nanofibers as Sustainable Microwave Absorbers

Avinash R. Paia, Binumol Tb, Deepu A. Gopakumarc, Daniel Pasquinid, Bastien Seantiere, Nandakumar Kalarikkala, Sabu Thomasa,b*

a International and Inter University Centre for Nanoscience and Nanotechnology,

Mahatma Gandhi University, Kottayam, Kerala-686560, India.

b School of Chemical Sciences, Mahatma Gandhi University, Kottayam, Kerala 686560, India.

c Université de Toulouse, IMT Mines Albi, RAPSODEE CNRS UMR-5302, Campus Jarlard, F-

81013 Albi Cedex 09, France.

d Chemistry Institute, Federal University of Uberlandia-UFU, Campus Santa Monica-Bloco1D-CP 593, Brazil.

e Univ. Bretagne Sud, UMR CNRS 6027, IRDL, F-56100 Lorient, France.

*Corresponding author: sabupolymer@yahoo.com, sabuthomas@mgu.ac.in

Graphical Abstract

Highlights:

- A 3D porous EMI shielding aerogel of cellulose nanofiber anchored poly(aniline) was fabricated.

- Shields exhibited maximum EMI shielding of -32 dB with a high specific EMI SE ~1667 dB.cm3.g-1.

- The porous shields predominantly absorbed (95%) excess EM radiations with minimal reflection (5%).

- These aerogels readily absorbed undesired EM radiations emitted out from a mobile phone.

- The aerogel possess ultra-fast heat dissipation behavior upon high power microwave exposure.

Abstract.

Electromagnetic (EM) pollution is ubiquitous and has soared to a great extent in the past few decades. The use of plant sourced cellulose nanofibers to fabricate sustainable and high performance electromagnetic shielding materials is foreseen as a green solution by the electronics industry to address this unseen pollutant. In this view, we report a facile and environmentally benign strategy to synthesize ultra-light and highly conductive aerogels derived from cellulose nanofibers (CNF) decorated with polyaniline (PANI) via a simple in-situ polymerization and subsequent freeze drying process devoid of any volatile organic solvents. The obtained conductive aerogels exhibited density as low as 0.01925 g/cc with a maximum EMI shielding value -32 dB in X band region. These porous shields demonstrated strong microwave absorption behavior (95%) with minimal reflection (5%) coupled with high specific EMI SE value ~1667 dB.cm3.g-1 which make these aerogels a potential candidate for use in telecommunication, military and defense applications.

Keywords: Cellulose Nanofibers, Sustainable Aerogels, Polyaniline, Electromagnetic Interference Shielding, Microwave Absorption, Heat Dissipation.