Photocatalytic reactor for methylene green degradation

Figure 2: Absorption (a) and fluorescence (b) spectra of MG (C = 10-5 M) with PMM before (1) and after 4 (2), 8 (3), 16 (4), 32 (5) and 64 (6) minutes of irradiation with KrCl excilamp at excitation wavelength 310 nm

Angelina Yurievna Petrova, TSU, Russia

Olga Nikolaevna Tchaikovskaya, Prof., TSU, Russia

Abstract (300 word limit)

Degradation of methylene green (MG) in photocatalytic reactor is studied. Water solutions of MG, of various concentrations in the presence of and without polypropylene microfiber material (PMM) modified with nanoparticles, are considered. The surface of PMM by IR spectra was investigated. IR spectra were obtained by a standard procedure using a Nicolet-380 spectrometer. The degradation of MG in water when exposed to UV radiation (KrCl excilamp, 222 nm) in the presence of PMM was studied from the absorption and fluorescence spectra. The absorption and fluorescence spectra with a CM2203 spectrophotometer (SOLAR, Belarus) were recorded.

The greatest optical density is observed after 4 and 8 minutes of irradiation (Figure 1a). With further irradiation, the optical density drops sharply, after which it again increases. This indicates the photodegradation of MG and the formation of photoproducts that absorb in the same region as the original molecule. With an increase in the concentration of MG in the solution to C=10-5 M, the greatest optical density is observed after irradiation for 60 minutes (Figure 2). The active formation of photoproducts in the fluorescence spectra after 16 minutes of irradiation (Figure 1b, curves 2-5) was recorded. As the concentration of MG in the aqueous solution increases, the formation of photoproducts increases (Figure 2b). Fixation of the formation of photoproducts, which adsorbed on the surface of the PMM.

An array of data on the photochemical properties of MG when exposed to UV radiation in photoreactor was obtained. Based on the analysis of the results obtained, it can be concluded that the polypropylene microfibrous material with a surface modified by nanoparticles actively participates in the photodegradation of MG in photocatalytic reactor. The results were obtained with the framework of the state task of the Ministry of Education and Science of the Russian Federation, project No. 4.6027.2017/8.9.

Image

Recent Publications (minimum 5)

- Kraiukhina V.S., Petrova A.Yu (2018). Investigation of photophysical processes occurring in methylene green. Proceedings, Proceedings of the Twenty-fourth All-Russian Scientific Conference of Students in Physics and Young Scientists (VNKSF-24, Tomsk).

- Petrova A.Yu., Tchaikovskaya O.N (2018). Fluorescence analysis of polyaromatic hydrocarbons photodegradation. Fluorescence analysis of polyaromatic hydrocarbons photodegradation. XVIIIth International Symposium on Luminescence Spectrometry, Brest, France, June 2018.

- Petrova A.Yu., Artjyshin V.R (2018). Degradation of methylene green in photocatalytic reactor. Materials of the XV International conference of students, graduate students and young scientists "Prospects for the development of fundamental sciences".

- Tchaikovskaya O., Karetnikova E., Sokolova I., Mayer G., Shvornev D. (2012). The phototransformation of 4-chloro-2-methylphenoxyacetic acid under KrCl and XeBr excilamps irradiation in water. J. Photochem. Photobiol. A: Chem. – 2012. – V. 228. – No. 1. – P. 8–14.

Figure 1: a - Absorption spectra of MG (C = 10-6 M) with PMM before (1) and after 4 (2), 8 (3), 16 (4), 32 (5) and 64 (6) minutes of irradiation with KrCl excilamp, b - Fluorescence spectra of MG with PMM before (1) and after 2 (2), 4 (3), 8 (4), 16 (5), 32 (6) and 64 (7) minutes of irradiation (at excitation wavelength 310 nm)

Gómez M., Murcia M.D., Gomez E., Ortega S., Sanchez A., Tchaikovskaya Olga N., Bryantseva Natalya G. (2016). Modelling and experimental checking of the influence of substrateconcentration on the first order kinetic constant in photo-processes J. of Environmental Management. 2016. Vol. 183, № 3. P. 818-825.

Biography (150 word limit)

Angelina Yurievna Petrova is postgraduate student of Faculty of Physics, TSU. She has her expertise in degradation of toxicants in photocatalytic reactor. Due to the addition of polypropylene microfiber material modified with nanoparticles, the degradation of the toxicant is more effective. Based on this data, a system is being developed that will allow the purification of water for reuse.

Email: petrovaayu@gmail.com

Biography (150 word limit)

Olga Tchaikovskaya is Professor of Faculty of  Physics, Tomsk State University. Currently, her scientific activity to carry out had as global objective the application of different technologies: photodegradation, advance oxygen process, to the elimination of some of the organic pollutants (phenol, chlorophenols, dyes, pesticides, polyaromatic hydrocarbons, psoralen and other aromatic compounds) that are present, generally, in the effluents of diverse industries: alimentary, refinery, textile, hospital and pharmacy, pulp and paper, and whose elimination by means of conventional treatments offers difficulties non solved in a satisfactory way up to now. Additionally, the corresponding kinetic studies were carried out and the respective models were elaborated, with the aim of having the fundamental knowledge that makes possible the later design of a combined process that maximizes the yield of elimination of these pollutants. Of special interest is the effect of humic and polypropylene microfiber on the loss of toxicants in the water.

Email: tchon@phys.tsu.ru

Notes/Comments: