# High Accuracy Optoelectronic Jaw Motion Dynamic Tracking System

Liang-Yuan Cheng1, Chih-Yu Hsu1,2, Tzung-Han Lin3, Shyh-Yuan Lee1,2 and Yuan-Min Lin1,2

Abstract (300 word limit)

There are numerous methods to trace the mandibular motion nowadays, such as graphical method, electromagnetic fields, ultrasonic devices and optoelectronic devices. In this study, we presented a novel high-accuracy marked optoelectronic jaw motion tracking system and related algorithms. Comparing to the popular ultrasonic-based JMA devices and other developed optoelectronic devices, which are heavy and can cause occlusal interference during tracking, our lightweight devices are optical-based detection system including a mandibular tracer, an eyewear facebow device, and stereo-vision cameras. Eyewear facebow and mandibular tracer provided information regarding the locations and relative movement of upper and lower jaws. Using OpenCV-based self-developed algorithm and post-iterative compensator, we were able to record the dynamic jaw movement in virtual environments with a high accuracy of mean error is 65.6 μm in x, y, and z directions, and our RMS errors are less than 101.2 μm at working distance. The lightweight devices had minimal occlusal disturbance and provides smooth motion-tracking experiences. The accuracy of our system is at least a quarter order better than other techniques, and it is relatively low-cost.

Image

Recent Publications (minimum 5)

- Shyh-Yuan Lee, Sheng-Chien Wu, Hsuan Chen, Lo-Lin Tsai, Jy-Jiunn Tzeng, Chih-Hsin Lin, and Yuan-Min Lin*, Synthesis and characterization of polycaprolactone-based polyurethanes for the fabrication of elastic guided bone regeneration membrane. Biomed Research International (Accepted)

- Jy-Jiunn Tzeng, Yi-Ting Hsiao, Yun-Ching Wu, Hsuan Chen, Shyh-Yuan Lee, and Yuan-Min Lin*, Synthesis, Characterization, and Visible Light Curing Capacity of Polycaprolactone Acrylate. Biomed Research International (Accepted)

- Chang, Wei-Ting; Hsu, Huang Jan; Jiang, Cho-Pei; Lee, Shyh-Yuan; Lin, Yuan-Min＊ (2017, Apr). Effects of Titanium dioxide and tartrazine lake dye on Z-axis resolution and physical properties of light-cured resin printed by a visible-light 3D printer . Rapid prototyping journal, 24(4). (Accepted)

- Chih-Hsin Lin, Ming-Chau Chang, Shih-Chieh Hung, Shyh-Yuan Lee, Yuan-Min Lin*, Bioactive surface modification of polycaprolactone using MG63-conditioned medium can induce osteogenic differentiation of mesenchymal stem cells , Journal of Materials Science, 2017. 52(7):p3967-3978 (IF=2.302, R/C= 82/271 in Materials science, Multidisciplinary)

- Yung‐Chang Cheng*, Deng‐Huei Lin, Cho‐Pei Jiang, Yuan‐Min Lin*, Dental implant customization using numerical optimization design and 3‐dimensional printing fabrication of zirconia ceramic, International Journal for Numerical Methods in Biomedical Engineering, doi: 10.1002/cnm.2820.

Biography (150 word limit)

Dr. Yuan-Min Lin, DDS, MSc, PhD, is an Associate Professor at the Department of Dentistry, National Yang-Ming University, Taiwan. He is the head of the Laboratory of Digital Dentistry and Dental Materials. Dr. Lin is also the founder of the Enlighten Materials company, a company focusing on the 3D printing and bioprinting materials..

Email: ymlin@ym.edu.tw