Graphene Nano-Platelets Embedded with Cellulose/Polypyrrole Composite for Di-electric Evaluation

Ahmed A. Haroun*, National Research Centre, Giza, Egypt.

A.M. Elnahrawy, National Research Centre, Giza, Egypt.

A.B. Abou Hammad, National Research Centre, Giza, Egypt.

S. Kamel, National Research Centre, Giza, Egypt.

### Abstract

Graphene nano-platelets (GNPs) were blended with cellulose and/or polypyrrole (PPy) to obtain composites via emulsion in combination with sonication strategy technique with ratio (1:1:1). The resulting composites were characterized using different analytical tools such as: Fourier transform infrared spectroscopy (FTIR), thermogravimetric analysis (TGA), transmitting electron microscope (TEM), and particle size distribution analysis using DLS technique. The results indicated that GNPs could be successful coated on cellulosic fibers in presence of epichlorohydrin as crosslinker and/or polypyrrole as conducting polymer. Besides, GNPs were dispersed in clusters shape inside the cellulose fibers with particle size about 155 nm. After adsorption of polypyrrole, the size was increased to around 429-460 nm. The results also exhibited that addition of GNPs increases the thermal stability of the cellulose/polypyrrole composite. Furthermore, dielectric properties, including constant (΄), dielectric losses (˝), loss tangent (tan δ) and ac conductivity were investigated over frequency range from 42 Hz to 5 MHz at different temperatures.

### Recent Publications

- El Nahrawy A M., Ahmed A. Haroun, Abou Hammad A B., Mohamed A. Diab, Kamel S (2018) Uniformly embedded cellulose/polypyrrole-TiO2 composite in Sol-gel sodium silicate nanoparticles: structural and dielectric properties. Accepted manuscript Silicon. doi.org/10.1007/s12633-018-9910-4.

- Amany M. ElNahrawya, Ahmed A. Haroun, Imad Hamadneh, Ammar H. Al-Dujailid, Samir kamel (2017) Conducting cellulose/TiO2 composites by in situ polymerization of pyrrole. Carbohydrate Polym 168: 182-190.

- Kumar A., Sarmah S (2011) AC conductivityand dielectric spectroscopic Studies of polypyrrole–titanium dioxide Solidi hybrid nanocomposites. Physica Status A, 208(9): 2203–2210.

- El-Sayed S., Mahmoud, K.H., Fatah A.A., Hassen A (2011) DSC, TGA and dielectric properties of carboxymethyl cellulose/polyvinyl alcohol blends. Physica B 406: 4068–4076.

- Pietro C, Athanassia A.,  Ilker S.B (2018) Graphene Nanoplatelets-Based Advanced Materials and Recent Progress in Sustainable Applications. Appl. Sci. 8: 1438; doi:10.3390/app8091438.

Figure 1. Schematic representation for preparation of embedded

GNPs with cellulose/PPy composite.

Ahmed A. Haroun currently works at the Division of Chemical Industries Research, National Research Centre, Giza, Egypt. He does research in Materials Chemistry, organic Chemistry, polymers chemistry and Nanotechnology. Their current projects deals with preparation, characterization of nano-materials structure, polymeric composites and their different applications in industrial and biomedical sectors. He authored/coauthored Over 55 peer-reviewed scientific articles published in international and national journals, besides, 2 national patents. He got several awards and scientific missions abroad and supervised about 10 Ph.D. and M.S. degree theses. He is a potential reviewer and member of editorial board of several international/national journals and scientific organization.

Email: haroun68_2000@yahoo.com

Notes/Comments:

The authors acknowledge the Science Development and Technology Fund (STDF), Egypt and Scientiﬁc (SRSF), Research Support Fund Jordan for ﬁnancial

Support of the research activities related project; ID 22929.