Techniques to Optimize the Exposure of Pediatric Patients on Adult Scanners

Abstract:

This report proposes techniques for optimizing the radiation exposure of pediatric patients to adult CT scanners in developing countries. A prospective method was carried out. Most hospitals in this country have older generations of adult CT scanners without a functioning pediatric (software) protocol. A study was done in 2015 and 2018 on 161 pediatric patients to
verify improved practices in the hospitals studied through the proposed strategies. In 2015, the principles of radioprotection were not rigorously respected, but in 2018, the practice improved. The sensitization of medical staff in terms of radioprotection today is visible. Dose parameters decreased in 2015 to 2018 for skull examinations. The activation of the pediatric protocol option in the software used for adult CT scanner in hospitals, the reduction of high tube voltage (kV), the tube current-time product (mAs) and the respect of the immobilization measures proposed in this work according to the ages are factors contributing to the improvement of pediatric examinations in this emerging country. The use of radiological protocols and procedures that are not suitable for pediatric examination delivers great scan lengths resulting in exposure to ionizing radiation of some unnecessary parts in an examination.
Keywords: Pediatric CT Scanner, Optimization of Protocols, Dose of Ionizing Radiation

Recent Publications

1-Protection of the Eyes, Thyroid and Gonads in Pediatric Tomodensitometry: Use of the Leaded Apron

2- Reduce Absorbed Doses and Protection of Radiosensitive Organs of Children Exposed to Ionizing Radiation on Adult Scanners

3- Techniques to Optimize the Exposure of Pediatric Patients on Adult Scanners

4- Dose Optimization in Computer Tomography Pediatric Cranial Scans

5- Optimization of pediatric CT scans in a developing country

6- Radiation dose evaluation of pediatric patients in CT brain examination: multi-center study.

7- Optimization of the scan length of head traumas on the pediatric and adult CT scan and proposition of a new acquisition limit