On Evaluation of Fracture Opening and Compressibility of Hydrocarbon Reservoirs

Martyushev D.A.1

1Oil and Gas Technology Department, Perm National Research Polytechnic University, Perm, Russia

Keywords: Carbonate reservoir, Indicator injection, Fracture opening, Fracture compressibility, well test

- Abstract Body

In this paper we compared among three methods of determining fracture: Warren-Root, Prof Victorin and tracer studies. Each of these methods has shown sufficiently reliable results and, therefore, any of them can be used to determine the fracture opening, which has a large effect on reservoir permeability. Method of tracer indicators allows us to study the real fluid flows between wells. Processing and interpretation of research data traffic tracer indicators makes it possible to quantitatively evaluate the distribution of these flows and define the parameters. This method has a major drawback: the long duration of the study (sometimes it can last up to 6 months). The empirical formula of Prof Victorin takes into account the initial opening of fracture and their compressibility, which can be determined using core samples and thin sections of rock. The disadvantages of this method are: (1) there is currently only about 25% of wells drilled with coring in Permsky kray, which is not sufficient for a comprehensive evaluation of fracture distribution; (2) the laboratory core analysis is not always possible to r estimate the fracture characteristics due to a possible surface cracking of the samples, which is often attributed to a small size of the samples. Methods Warren and Root are based on the result of the pressure recovery curves. The disadvantage of this method is that for the evaluation with this method, the well production must be stopped. The selection of an appropriate method, therefore, will depend on the resources, time available for the evaluation and economic objectives of the oil company.

- Image: Well locations in carbonate reservoirs

- Reference:

- Martinez N.R., Samaniergo F.V. Advances in analysis of pressure interference tests // Journal of Canadian Petroleum Technology. 2010. Vol.49. №12. Pp.65-70

- Chang C., Zoback M.D., Khaksar A. Empirical relations between rock strength and physical properties in sedimentary rocks // Journal of Petroleum Science and Engineering. 2006. Vol.51. Pp.223-237.

- Raghavan R., Chen C. Fraction diffusion in rocks produced by horizontal wells with multiple, transverse hydraulic fractures of finite conductivity // Journal of Petroleum Science and Engineering. 2013. Vol.109. Pp.113-143

- Biography:

Martyushev D. has completed his PhD at the age of 28 years from Perm National Research Polytechnic University (PNRPU), Perm, Russia. He is the senior lecturer of the Department of Oil and Gas Technology PNRPU and researcher Research and Education center of Geology and Development of Oil and Gas Fields. He has published more than 20 papers in reputed journals.

- Presenting author details

Full Name: Martyushev Dmitriy A.

Contact number: 8-951-922-79-88

Twitter account: -

Linked In account: -

Session name/ number: Reservoir Engineering and Reservoir Simulation

Category: Oral presentation