Title: Impact of connate water and water salinity on relative permeability of carbonate reservoirs in Enhanced Oil Recovery Methods

### Samira Mohammadkhani 1 Hamidreza Shahverdi 1,  Mohsen Nasr Esfahany 1

1 Department of Chemical Engineering, Isfahan University of Technology, Isfahan 84156-83111, Iran

### Keywords: Enhanced Oil recovery, Low salinity, relative permeability, Carbonate rock

- Abstract Body (up to 250 words)

The salinity and saturation of the connate water as well as the injected water salinity are among the important parameters that affect the oil recovery during low salinity water injection (LSWI). One of the main mechanisms occurring in this process is wettability alteration that significantly modifies the relative permeability (kr) curves. The purpose of this study is to derive the relative permeability curves of the core-flood experiments performed under various conditions of LSWI.

First, five core flooding experiments were conducted using limestone core samples. The seawater and various diluted seawater were used as injection water. The initial water saturation in the cores were also varied in each experiment to sensitize the impact of initial saturation on the performance of LSWI and kr curves.

The kr curves of each tests were estimated by history matching the core-flood tests. In this procedure the parameters of kr curves (LET correlation) were tuned in an optimization algorithm. In other words, the numerical simulator was linked to the Ant-colony (as optimization method), then the kr curves were changed in an iterative manner to get a reasonable match between experiment and simulation data. The comparison between kr curves of various tests depict the wettability alteration during LSWI and highlights the impact of salinity of injection water.

### Image:

Reference:

- Mohammadkhani, S, S Marie Nielsen, A Shapiro, H Shahverdi, and M Nasr Esfahany. 2018. "The Potential of Temperature-activated CO2 Production In Situ as an Enhanced Oil Recovery Process." 80th EAGE Conference and Exhibition 2018.

- Mohammadkhani, Samira, Hamidreza Shahverdi, and Mohsen Nasr Esfahany. 2018. "Impact of salinity and connate water on low salinity water injection in secondary and tertiary stages for enhanced oil recovery in carbonate oil reservoirs."  Journal of Geophysics and Engineering 15 (4):1242-1254.

- Yaralidarani, Muhammad, and Hamidreza Shahverdi. 2016. "An improved Ant Colony Optimization (ACO) technique for estimation of flow functions (kr and Pc) from core-flood experiments."  Journal of Natural Gas Science and Engineering 33:624-633.

- Zaeri, Mohammad Reza, Rohallah Hashemi, Hamidreza Shahverdi, and Mehdi Sadeghi. 2018. "Enhanced oil recovery from carbonate reservoirs by spontaneous imbibition of low salinity water."  Petroleum Science 15 (3):564-576.

- Zaeri, Mohammad Reza, Hamidreza Shahverdi, Rohallah Hashemi, and Mohsen Mohammadi. 2018. "Impact of water saturation and cation concentrations on wettability alteration and oil recovery of carbonate rocks using low-salinity water."  Journal of Petroleum Exploration and Production Technology:1-12.

### Biography:

Hamidreza Shahverdi is an assistant professor in Isfahan University of Technology, at department of Chemical Engineering. He is head of petroleum department at university. He has 15 years of experiences in petroleum reservoir engineering in particular in EOR techniques and multi-phase flow in porous media. He is consultant of EOR project at Heriot-watt university. Shahverdi holds a PhD degree in petroleum engineering from Heriot-Watt University (at UK).

### Presenting author details

Full Name: Hamidreza Shahverdi.

Contact number: +98 (0)9134033465, +98 (031)33915603

Email: Hr_Shahverdi@cc.iut.ac.ir

Category: (Oral presentation/ Poster presentation)

Passport Number: B43559407