CO2 Minimum Miscibility Pressure (MMP) Correlation predicts Slim tube Experimental Data very precisely

Osamah A. Alomair, PhD, Kuwait University

Maqsood Iqbal, MSc, Kuwait University

Abstract

Miscible gas injection is one of the effective enhanced oil recovery (EOR) method.  MMP  is the most important parameter to successfully design CO2 flooding, the local displacement efficiency from gas injection is also highly MMP dependent, which is traditionally measured through time consuming, expensive and cumbersome experiments.  A new CO2 MMP correlation based on multiple-linear-regression modeling technique has been developed to more accurately estimate the CO2 MMP for a wide range of live and heavy crude oils.  The proposed CO2 MMP correlation is originated from CO2 MMP experimental data in addition to database from the worldwide published literature that covers 33 pure CO2 MMP data for various live and dead oil samples.  The proposed model is trained by exploiting 66% (22 data points) of the data bank.  This correlation is expressed as a function of reservoir temperature, mole fractions of in situ reservoir CO2 along with two its hydrocarbon components (C1 and C3).  A set of experimental data pool from the literature is collected to evaluate and compare the results of the developed correlation with pre-existing correlations through statistical and graphical error analyses.  A statistical comparison is performed for both training data set (22 data points) as well as testing data set (11 data points).  It is found that the proposed CO2 MMP correlation provides the best reproduction of MMP data with a percentage average absolute error of 11.46 % and 11.92 % for training and testing categories respectively.  Further the correlation coefficient for the proposed correlation for training and testing data are 0.957 and 0.929 respectively.  Finally, by employing the relevancy factor, it is found that the after temperature both light and intermediate (C1 and C3) components of crude oil have the most significant impact on the CO2 MMP estimation.

Recent Publications

- Osamah A. Al-Omair, Salah M. Al-udhhi, and Mabkhout M. Al-Dousari‏, "Investigating of Rock-Face Boundary Effects on Capillary Pressure and Relative Permeability Measurements", Journal of Porous Media, Vol. 14 (5), pp 395-409, 2011.

- O, Alomair, A. Malallah, A. Elsharkawy, and M. Iqbal, "Predicting CO2 Minimum Miscibility Pressure (MMP) Using Alternating Conditional Expectation (ACE) Algorithm", Oil & Gas Science and Technology, Accepted for publication on December 2012, Published online in June 2013.

- Alomair Osamah and Almusallam Abdulwahab,  "Heavy  Crude Oil Viscosity Reduction and the Impact of Asphaltene Precipitation", Energy & Fuels, Vol. 27 (12), pp 7267-7276, December, 2013.

- Osamah Alomair, Adel Elsharkawy, Hassan Alkandari, "A Viscosity Prediction Model for Kuwaiti Heavy Crude Oils at Elevated Temperatures", Journal of Petroleum Sciences and Engineering, Vol. 120, pp 102-110, August 2014.

- Osamah Alomair, Mohammed Jumaa, Abdulhaq Alkoriem, and Mohamed Hamed, "Heavy oil viscosity and density prediction at normal and elevated temperatures", Journal of Petroleum Exploration and Production Technology, Vol. 6 (2), pp 253-263, June, 2016.

- Osamah Ali Alomair, Khaled M Matar, Yousef H Alsaeed, "Experimental Study of Enhanced-Heavy-Oil Recovery in Berea Sandstone Cores by Use of Nanofluids Applications", SPE Reservoir Evaluation & Engineering, Vol 18 (3), August 2015.

- Osamah A. Alomair and Ali A. Garrouch, "A general regression neural network model offers reliable prediction of CO2 minimum miscibility pressure", Journal of Petroleum Exploration and Production Technology, Vol. 6 (3), pp 351-365, September, 2016.

Biography

Osamah A. Alomair is an Associate Professor in the Petroleum Engineering Department at Kuwait University. He has been with Kuwait University since 2001.  Alomair's current interests include EOR, application of Nano technology, fluid behaviors, and asphaltene studies. He holds a PhD degree in petroleum engineering from Colorado School of Mines.

Email:   osamah_71@hotmail.com

Maqsood Iqbal is a Senior Reservoir Engineer with  20 years of diversified experience; skills encompass Reservoir Engineering, enhanced oil recovery (/IOR), Production Operations  /Optimization and Petroleum Economics. Exclusive aptitudes; Well Test Analysis (WTA), Decline Curve Analysis (DCA), Material Balance Calculations, Temperature Logs Interpretations, Pressure-Volume-Temperature (PVT) studies, Non-Parametric Regression and Artificial Neural Networks.  He holds M. Sc. Petroleum Engineering degree from Kuwait University.

Email:    maqsoodqp@hotmail.com