Title: 3D Modeling of Dynamic Interaction of Steel Building Structures on Raft Foundation

### Kant  L.1,  Samanta A.1 , Saurabh S.1,

1 Department of civil and environmental engg., Indian Institute of technology Patna, India

### Keywords: SMRF, raft foundation, soil-structure-interaction, storey height, nonlinear soil.

- Abstract

Structural design engineers often ignore the effect of soil-structure interaction for low storey buildings. They consider these effects only for heavy structures. Soil- structure interaction (SSI) phenomenon can also be extended to the adjacent structures. Dynamic effects of proximity of buildings in a congested area will be very critical in its design. Very few research conducted in this fields, were limited to lumped mass model or two dimensional (2D) finite element method with plain-strain consideration.2D modeling with plain stress-stress assumptions may lead to erroneous results. In this research, in order to account the effects of proximity on the response of adjacent buildings under seismic excitation, three dimensional(3D) models FEM models of 4 and 6 storey  steel moment-resisting frame (SMRF) building structures on raft foundation are used. The soils beneath foundation are clay and sand. Dynamic analyses are used under several actual earthquakes taken from PEER Ground Motion Database - PEER Center. There are three group of combinations of two adjacent SMRF buildings are considered, i) two 4-stories, ii) two 6-stories, and iii) one 4- and one 6-stories.This study are based on direct method of modeling for dynamic SSI. Mohr’s column soil behavior is assumed in this study while structures are assumed to behave in elastic-plastic manner. Results show that the seismic response of adjacent buildings structures depends on structural heights, type and depth of the foundation soil and the distance between the adjacent buildings.

- Reference:

1	Jeng V, Tzeng WL. Assessment of seismic pounding hazard for Taipei city. Engineering Structures 2000;

22(5):459–471.

- Luco JE, Contesse L. Dynamic structure-soil-structure interaction. Bulletin of the Seismological Society of America

1973; 63(4):1289–1303.

- Chouw N. Influence of soil-structure interaction on pounding response of adjacent buildings due to near-source earthquakes.Journal of Applied Mechanics 2002; 5:543–553.

### Biography:

Lakshmi Kant is currently pursuing a Ph.D. degree from Indian Institute of Technology Patna in the department of Civil and Environmental Engineering, specializing in structural Engineering. Before starting his Ph.D., he received his Master’s degree from the Indian Institute of Technology Roorkee in 2014. His research is a numerical modelling of the soil-structure interaction and its application to design of structure and assessment of seismic hazards.  He is currently working on a FEM simulation of the Soil-Structure interaction.  His research is expected to provide a powerful decision making tool to be used by structural engineers for design of ‘Smart Infrastructure against seismic hazards'.

### Presenting author details

Full Name: Lakshmi Kant

Contact number:+91-9155746298

Mail Id.lakshmi.pce14@iitp.ac.in

Category: (Oral presentation)

Passport Number: N9298520

Copyright © 2016-2018 Meetings Int PTE Ltd, All Rights Reserved.

Meetings Int.Pte Ltd

## Company Registration No: 201524282R,28 MAXWELL ROAD, #03-05 RED DOT TRAFFIC, SINGAPORE (069120)

Tel:; +6531080483| Toll Free no. +800-852-6126

Web:  https://www.meetingsint.com/conferences/structuraldesign | E-mail: structuraldesign@annualmeetings.net