# Abstract Template

Title: Vulnerability Assessment for RC building with Vertical Irregularity in India

### Saurabh  S.1,  Samanta A.1 , Pratyush k.1

1 Department of civil and environmental engg., Indian Institute of technology Patna, India

### Keywords: Fragility curves, Intermediate soft storey, Open ground storey, Pushover analysis.

- Abstract

Seismic hazards cause massive losses to structures, so the assessment of its health is important for the rehabilitation. This assessment is done in terms of probability of exceeding certain damage states, termed as fragility analysis or vulnerability assessment. Reinforced concrete (RC) buildings with infill masonry construction are found to have resisted more lateral loads and have less nodal drift with respect to bare frames. But owing to achieve large floor area and to gain good aesthetic view, openings are prevalently practiced these days which ultimately leads to the creation of vertical irregularity in the form of open ground storey (OGS), intermediate soft storey, weak storey and even mass irregularity in RC buildings. In the presence of the above mentioned vertical irregularities, the roof drift value is relatively higher due to significant reduction in the stiffness, but the base shear that it attracts reduces significantly. In the present study, simplified performance assessment of RC frame buildings with varied infill configurations and varied material type is performed using pushover analysis, which is a nonlinear static analysis. The modeling of infill walls is done as an equivalent diagonal struts in SAP 2000, considering it as a multilinear plastic link elements having same axial stiffness as of infill walls. Considering the different variability in the model arising due to the material as well as vertical irregularities, fragility analysis is done. This study contributes towards the study of the vulnerability of soft storey RC buildings under different damage states.

.

- Reference:

- FEMA. NEHRP Guidelines for the seismic rehabilitation of buildings. FEMA 273, Washington, DC;1997.

- Borzi B., Pinho R., Crowley H. (2008). Simplified Pushover-Based Vulnerability Analysis for Large Scale Assessment of

RC Buildings, Engineering Structures, 30:3, 804-820.

- IS 1893:2016, criteria for earthquake resistant design of structures part 1 general provisions and buildings.

### Biography:

Saurabh Suman is currently pursuing a PhD degree from Indian Institute of Technology Patna in the department of Civil and Environmental Engineering, specializing in structural Engineering under the supervision of Dr.Avik Samanta. He received Graduation degree from the Jadavpur University,India in 2017. His research comprises of vulnerability assessment of building stocks. He is currently working on a Finite Element analysis of the buildings with irregularities. His research is expected to be useful in decision making for rehabilitation or retrofitting of damaged structures due to seismic hazards, and prepare for adequate facilities in chaotic situations.

### Presenting author details

Full Name: Saurabh Suman

Contact number:+91-7982653569

Mail Id.saurabh.pce17@iitp.ac.in

Category: (Oral presentation)

Passport Number: P0659890

Copyright © 2016-2018 Meetings Int PTE Ltd, All Rights Reserved.

Meetings Int.Pte Ltd

## Company Registration No: 201524282R,28 MAXWELL ROAD, #03-05 RED DOT TRAFFIC, SINGAPORE (069120)

Tel:; +6531080483| Toll Free no. +800-852-6126

Web:  https://www.meetingsint.com/conferences/structuraldesign | E-mail: structuraldesign@annualmeetings.net