GEOMETRIC DISTORTION AND STRESS REDISTRIBUTION OF BEAM-COLUMN STEEL CONNECTIONS UNDER PROGRESSIVE COLLAPSE SCENARIO

David Stephen1, Dennis Lam2, John Forth3, Jianqiao Ye4, Konstantinos Daniel Tsavdaridis5                                           1, 3, 5 School of Civil Engineering, University of Leeds, LS2 9JT

2School of Engineering design and Technology, Bradford, BD7 1DP

4Lancaster University Engineering Building, LA1 4YR

ABSTRACT

This paper investigates the influence of catenary and shear force developed during progressive collapse on steel beam-column connections. The overall performance of a beam-column connection under abnormal loading condition is crucial to the stability of the structure to progressive collapse. BS EN 1993-1-8 requires that connections are classified based on their stiffness or strength. These two important parameters depend on the complex internal forces interaction mechanism with the geometric shape of the components (beam, column, plates, bolts and nuts) and material properties of the components. However, connection designs often assume either a pin or rigid behaviour despite the behaviour is semi-rigid. Moreover, it is argued that large catenary forces are developed in beams could trigger global instability of the structural system under progressive collapse scenario. A finite element modelling software ABAQUS is used to examine a validated model of the connection for progressive collapse.  The results show that the maximum plastic strain of the bolts due to catenary forces is 23.9%. At the column web, a maximum plastic strain of 19.6% is characterised by geometric instability because of distortions. These distortions are due to the column flanges bulging outwardly on one side and inwardly on the opposite side thereby triggering a progressive collapse.

Keywords: Abnormal loads, progressive collapse, catenary actions, beam-column connection, Structures, column removal, ABAQUS (FEA)

.